Table of Contents

# 工种工艺

各工种施工工艺详解，从入门到精通。

## 目录

- 概述 — 工种工艺总览

- 水电工工艺 — 水路电路布线施工

- 泥瓦工工艺 — 砌筑、贴砖、防水

- 木工工艺 — 吊顶、隔墙、定制柜

- 油漆工工艺 — 墙面处理、涂料、壁纸

- 安装工工艺 — 地板、门、洁具安装

# 厨房防溅墙工艺 (Kitchen Backsplash Craft)

## 概述

厨房防溅墙（Backsplash）是操作台后方的墙面保护系统，主要功能是防止烹饪时的油污、水渍和水蒸气侵蚀墙面。同时，防溅墙也是厨房设计中的重要视觉元素，影响整体装修风格。本指南涵盖材料选择、安装工艺和维护保养。

## 材料选项对比

### 材料综合对比

| 材料 | 价格（元/m2） | 防污性 | 耐热性 | 施工难度 | 美观度 | 使用寿命 |
| --- | --- | --- | --- | --- | --- | --- |
| 釉面瓷砖 | 50–300 | 优 | 优 | 中 | 高 | 20 年以上 |
| 马赛克 | 100–800 | 良 | 优 | 高 | 极高 | 20 年以上 |
| 钢化玻璃 | 200–800 | 极佳 | 良 | 低 | 高 | 10–15 年 |
| 不锈钢 | 150–500 | 极佳 | 极佳 | 低 | 中 | 15–20 年 |
| 天然石材 | 300–2000+ | 中（需密封） | 优 | 高 | 极高 | 20 年以上 |
| 人造石 | 300–1500 | 优 | 良 | 中 | 高 | 15–20 年 |
| 防火板 | 100–300 | 良 | 中 | 低 | 中 | 5–10 年 |
| 铝塑板 | 80–200 | 良 | 中 | 低 | 中 | 8–12 年 |
| 珐琅板 | 500–2000 | 极佳 | 极佳 | 中 | 高 | 20 年以上 |

### 各材料详解

#### 1. 釉面瓷砖

优点： - 性价比高，选择丰富 - 耐油污、易清洁 - 耐高温、耐化学腐蚀 - 使用寿命长

缺点： - 接缝处易积污（需要定期清洁美缝） - 施工周期较长

推荐规格：

| 规格（mm） | 适用场景 | 美缝建议 |
| --- | --- | --- |
| 300x300 | 经济型、小厨房 | 环氧美缝 |
| 300x600 | 主流选择 | 环氧美缝 |
| 400x800 | 大厨房、现代风格 | 环氧美缝 |
| 600x600 | 大面积防溅墙 | 环氧美缝 |
| 600x1200 | 高端、减少接缝 | 环氧美缝 |

#### 2. 钢化玻璃

优点： - 无接缝，彻底杜绝缝隙污染 - 极易清洁（一擦即净） - 可定制图案和颜色 - 安装快捷

缺点： - 不耐尖锐物体撞击 - 长期使用可能出现划痕 - 表面水渍较明显 - 不可现场切割

规格建议：

| 参数 | 要求 |
| --- | --- |
| 玻璃厚度 | 5–6mm 钢化玻璃 |
| 安全认证 | 必须有 3C 认证标志 |
| 边缘处理 | 精磨边，防止割手 |
| 背漆 | 背涂颜色，正面为透明钢化玻璃 |

#### 3. 不锈钢

优点： - 商用厨房标准，耐油污极佳 - 耐高温、耐冲击 - 一体成型无接缝 - 可磁性吸附厨具

缺点： - 容易产生划痕 - 风格偏工业/商用 - 水渍和指纹明显 - 可能产生共振噪音

规格建议：

| 参数 | 要求 |
| --- | --- |
| 材质 | SUS304 不锈钢 |
| 厚度 | 0.8–1.2mm |
| 表面处理 | 拉丝（推荐）或镜面 |
| 边缘 | 折边处理 |

#### 4. 天然石材

优点： - 质感奢华 - 纹理独一无二 - 耐高温

缺点： - 必须定期密封 - 酸性物质（柠檬汁、醋）会腐蚀 - 价格昂贵 - 接缝需专业处理

推荐品种：

| 品种 | 特点 | 注意事项 |
| --- | --- | --- |
| 大理石 | 纹理优美 | 需密封，避免接触酸性物质 |
| 花岗岩 | 硬度高、耐腐蚀 | 需确认放射性达标 |
| 洞石 | 独特孔洞纹理 | 孔洞需填补密封 |
| 石英石 | 硬度极高、零渗透 | 人造石材，性能优异 |

## 安装高度标准

### 常规高度

| 区域 | 标准高度 | 最低高度 | 推荐高度 |
| --- | --- | --- | --- |
| 灶台后方 | 700–800mm | 600mm | 800mm（至吊柜底） |
| 水槽后方 | 400–500mm | 300mm | 500mm |
| 操作台其他区域 | 300–400mm | 200mm | 400mm |
| 通高防溅墙 | 台面至吊柜底/天花板 | – | 整体视觉效果佳 |

### 高度计算

灶台防溅墙高度 = 吊柜底标高 - 台面标高

例如：
  吊柜底标高：1550mm
  台面标高：850mm
  防溅墙高度 = 1550 - 850 = 700mm

### 宽度范围

| 区域 | 宽度范围 |
| --- | --- |
| 灶台区域 | 灶具宽度 + 两侧各 150–300mm |
| 水槽区域 | 水槽宽度 + 两侧各 100–200mm |
| 整排台面 | 橱柜总长度 |

## 插座与开关处理

### 插座定位原则

| 原则 | 说明 |
| --- | --- |
| 高度 | 距台面 300–400mm |
| 水平间距 | 每 800–1200mm 设置一个插座 |
| 灶具上方 | 严禁设置插座 |
| 水槽附近 | 设置带防溅盖的插座 |
| 数量 | 至少 3–5 个（电饭煲、微波炉等） |

### 防溅墙开孔工艺

#### 瓷砖开孔

| 步骤 | 操作 |
| --- | --- |
| 1. 定位 | 按实际插座位置在瓷砖上标记 |
| 2. 开孔 | 使用玻璃开孔器或瓷砖开孔器 |
| 3. 孔径 | 比插座面板大 3–5mm |
| 4. 安装 | 插座面板盖住开孔边缘 |

#### 玻璃/石材开孔

| 步骤 | 操作 |
| --- | --- |
| 1. 工厂开孔 | 必须在工厂按图纸开孔（现场无法加工） |
| 2. 孔位精度 | 偏差 +/- 2mm |
| 3. 边缘处理 | 孔边精磨 |
| 4. 安装 | 插座面板盖住孔边缘 |

### 插座周围密封

| 方法 | 适用材料 | 操作步骤 |
| --- | --- | --- |
| 硅胶密封 | 所有材料 | 插座面板四周打一圈中性硅胶 |
| 橡胶垫圈 | 玻璃/不锈钢 | 面板背面加橡胶密封垫 |
| 专用密封盖 | 瓷砖 | 使用防水插座盖 |

## 施工工艺流程

### 瓷砖防溅墙施工

#### 步骤一：基层处理

- 检查墙面平整度（3mm/2m 以内）

- 清理墙面浮灰、油污

- 涂刷墙固/界面剂

#### 步骤二：弹线排版

- 弹出水平基准线和垂直控制线

- 根据瓷砖规格排版

- 确定非整砖位置（尽量放在不显眼处）

- 确定插座开孔位置

#### 步骤三：铺贴

- 使用瓷砖胶（C2 级）薄贴

- 双面刮胶（墙面和瓷砖背面）

- 用齿形刮板刮出条纹

- 揉压粘贴

- 用十字卡控制接缝宽度（1.5–2mm）

#### 步骤四：美缝

- 铺贴完成 48 小时后进行

- 清理缝隙（深度不小于 2mm）

- 用环氧美缝剂填缝

- 压平、擦拭多余材料

#### 步骤五：收边

| 位置 | 收边方式 |
| --- | --- |
| 顶部 | 与吊柜收口，打硅胶或安装收边条 |
| 底部 | 与台面收口，打防霉硅胶 |
| 侧面 | 阳角 45 度碰角或阳角条 |
| 插座 | 插座面板四周密封 |

### 玻璃防溅墙安装

- 工厂按尺寸定制加工（含插座开孔）

- 墙面找平处理

- 用结构胶/玻璃胶粘贴上墙

- 底部打防霉硅胶收口

- 插座面板安装并密封

- 24 小时后使用

### 不锈钢防溅墙安装

- 工厂按尺寸定制加工（含折边和开孔）

- 墙面找平处理

- 用结构胶粘贴上墙

- 或用膨胀螺丝固定（孔帽装饰）

- 四周打密封胶收口

## 接缝处理

### 美缝材料选择

| 材料 | 防污性 | 防霉性 | 耐久性 | 价格（元/m） | 推荐度 |
| --- | --- | --- | --- | --- | --- |
| 白水泥 | 差 | 差 | 差 | 1–2 | 不推荐 |
| 填缝剂 | 一般 | 一般 | 一般 | 3–5 | 不推荐 |
| 环氧美缝剂 | 优 | 优 | 优 | 10–20 | 强烈推荐 |
| 聚脲美缝剂 | 极佳 | 极佳 | 极佳 | 15–30 | 高端推荐 |
| 硅胶填缝 | 良 | 良 | 一般 | 5–10 | 局部使用 |

### 接缝宽度

| 瓷砖规格 | 推荐缝宽 | 说明 |
| --- | --- | --- |
| 小砖（< 100mm） | 1–1.5mm | 仿古风格可加宽 |
| 中砖（300–600mm） | 1.5–2mm | 标准接缝 |
| 大砖（> 600mm） | 2–3mm | 留缝过小易空鼓 |
| 密缝铺贴 | 0.5–1mm | 需瓷砖精度极高 |

## 清洁与维护

### 日常清洁

| 材料 | 清洁方法 | 禁忌 |
| --- | --- | --- |
| 釉面瓷砖 | 中性清洁剂 + 软布 | 禁用强酸强碱 |
| 玻璃 | 玻璃清洁剂 + 刮水器 | 禁用钢丝球 |
| 不锈钢 | 不锈钢专用清洁剂 | 禁用含氯清洁剂 |
| 天然石材 | 石材专用清洁剂 | 禁用酸性清洁剂 |
| 人造石 | 中性清洁剂 | 禁用研磨性清洁剂 |

### 深度清洁周期

| 项目 | 周期 | 方法 |
| --- | --- | --- |
| 美缝清洁 | 每 3–6 个月 | 美缝清洁剂 + 软刷 |
| 石材密封维护 | 每 1–2 年 | 重新涂刷密封剂 |
| 不锈钢抛光 | 每半年 | 不锈钢保养剂 |
| 玻璃除水垢 | 按需 | 白醋或水垢清除剂 |
| 硅胶更换 | 每 2–3 年 | 铲除旧胶重新打胶 |

### 日常防护建议

- 烹饪时开启抽油烟机，减少油烟附着

- 使用后及时擦拭，避免油污固化

- 定期清理灶台前方区域，防止油污溅到防溅墙

- 避免将热锅直接靠在玻璃或人造石防溅墙上

- 石材防溅墙避免接触柠檬汁、醋等酸性物质

## 选材建议

### 按厨房风格选材料

| 风格 | 推荐材料 | 颜色建议 |
| --- | --- | --- |
| 现代简约 | 大规格瓷砖、玻璃 | 白色、灰色、黑色 |
| 北欧风格 | 小白砖（地铁砖）、大理石纹砖 | 白色、浅灰 |
| 工业风格 | 不锈钢、仿水泥砖 | 灰色、金属色 |
| 中式风格 | 仿大理石瓷砖、天然石材 | 米色、棕色纹理 |
| 美式风格 | 花砖、马赛克 | 暖色调、拼花 |
| 轻奢风格 | 大理石纹大砖、岩板 | 白色底 + 金色纹理 |

### 按预算选材

| 预算（含人工） | 推荐方案 |
| --- | --- |
| 500 元/m2 以下 | 常规釉面瓷砖 + 环氧美缝 |
| 500–1000 元/m2 | 大规格瓷砖/钢化玻璃 + 环氧美缝 |
| 1000–2000 元/m2 | 天然石材/珐琅板/岩板 |
| 2000 元/m2 以上 | 高端天然石材/定制岩板/一体成型台面延伸 |

## 施工检查清单

### 施工前

- 墙面平整度符合要求（3mm/2m）

- 插座位置已确定并预埋到位

- 材料已进场并检验合格

- 排版方案已确认

- 开孔位置和尺寸已确认

### 施工中

- 瓷砖胶/粘接材料使用正确

- 铺贴平整、无空鼓

- 接缝宽度均匀

- 开孔位置准确、边缘整齐

- 插座四周密封到位

### 施工后

- 平整度检测合格（2mm/2m）

- 空鼓检查合格

- 美缝饱满、无遗漏

- 收边收口美观

- 表面无划痕、污渍

- 所有插座功能正常

# 阳台装修指南

## 阳台类型

| 类型 | 特点 | 装修重点 |
| --- | --- | --- |
| 封闭式 | 可做成多功能空间 | 保温、隔音 |
| 开放式 | 通风好、视野好 | 防水、防晒 |

## 阳台功能

### 生活阳台

- 洗衣机柜

- 晾衣架

- 拖把池

### 休闲阳台

- 绿植花架

- 休闲桌椅

- 小水景

### 工作阳台

- 储物柜

- 工作台

- 工具收纳

## 装修要点

### 防水

- 地面做防水

- 墙面防水高度≥30cm

- 闭水试验必须做

### 材料选择

- 地砖：防滑、耐候

- 墙面：外墙漆或瓷砖

- 吊顶：铝扣板或防水石膏板

### 水电预留

- 洗衣机插座（带防溅盒）

- 水龙头

- 地漏

- 照明

### 封阳台

- 断桥铝窗户

- 双层中空玻璃

- 注意物业审批

## 常见问题

| 问题 | 原因 | 处理方法 |
| --- | --- | --- |
| 渗水 | 防水没做好 | 重做防水 |
| 漏风 | 窗户密封差 | 更换密封条 |
| 冷凝水 | 保温不好 | 加装保温层 |

# 卫生间施工工艺详解

## 防水施工

### 施工流程

- 基层处理

- 管根加强

- 底涂

- 面涂2-3遍

- 闭水试验

- 保护层

### 关键指标

- 防水层厚度≥1.5mm

- 闭水24小时无渗漏

- 淋浴区防水高度≥1.8m

## 贴砖施工

### 地面

- 防滑砖

- 向地漏找坡1-2%

- 规格300×300mm

### 墙面

- 釉面砖

- 规格300×600mm

- 墙砖压地砖

## 洁具安装

### 马桶

- 法兰圈密封

- 打胶固定

### 花洒

- 混水阀距地90-100cm

- 冷热水口间距15cm

### 浴室柜

- 壁挂式易清洁

- 落地式储物空间大

## 验收要点

- 防水闭水试验合格

- 地面无积水

- 排水顺畅

- 洁具安装牢固

- 不渗漏

# 卫生间装修指南

## 卫生间布局

### 基本功能区

- 马桶区：宽度≥80cm

- 洗漱区：宽度≥60cm

- 淋浴区：最小 80×80cm

### 干湿分离

- 淋浴区用隔断分离

- 马桶和洗漱区在外侧

- 小户型用浴帘+挡水条

## 防水施工

### 防水范围

- 地面全部

- 墙面：淋浴区≥1.8m，其他区域≥0.3m

- 管根、阴阳角加强处理

### 闭水试验

- 蓄水深度≥20mm

- 蓄水时间≥24小时

- 检查楼下对应位置

## 材料选择

### 地砖

- 防滑砖

- 规格 300×300mm

- 向地漏找坡 1-2%

### 墙砖

- 釉面砖

- 规格 300×600mm

- 颜色与地砖搭配

### 吊顶

- 铝扣板集成吊顶

- 防潮、易检修

## 水电预留

### 电路

- 镜前灯

- 智能马桶插座

- 电热毛巾架插座

- 吹风机插座

- 所有插座带防溅盒

### 水路

- 冷热水口

- 马桶进水

- 预留小厨宝位置

## 通风

- 排风扇风量≥100m³/h

- 有外窗优先自然通风

- 无外窗必须装排风扇

## 常见问题

| 问题 | 原因 | 预防方法 |
| --- | --- | --- |
| 渗水 | 防水没做好 | 闭水试验严格 |
| 反味 | 下水管没存水弯 | 装存水弯 |
| 积水 | 坡度不够 | 找坡1-2% |

# 卧室装修指南

## 布局设计

### 床的摆放

- 床头靠实墙

- 两侧留通道≥60cm

- 避免正对门

### 衣柜设计

- 深度60cm

- 到顶设计

- 内部挂衣区+叠放区+抽屉

## 色彩搭配

| 风格 | 主色 | 辅助色 | 点缀色 |
| --- | --- | --- | --- |
| 现代简约 | 白色 | 灰色 | 蓝色 |
| 北欧 | 浅灰 | 原木色 | 绿色 |
| 轻奢 | 灰色 | 黑色 | 金色 |

## 灯光设计

- 主灯：吸顶灯，暖白色

- 床头灯：壁灯或台灯

- 衣柜灯：感应灯带

## 材料选择

- 地板：实木复合或实木

- 墙面：乳胶漆或壁纸

- 窗帘：遮光70-90%

## 环保要求

- 板材选择ENF级

- 涂料选择低VOC

- 装修后通风3个月以上

# 定制柜安装工艺 — Custom Cabinet Installation Craft

## 1. Overview

Custom cabinet installation is a multi-stage process that begins with precise site measurement and extends through production, delivery, on-site assembly, leveling, door adjustment, hardware mounting, and final finishing. This guide covers the complete workflow for residential built-in cabinets including wardrobes, kitchen cabinets, TV units, and storage systems.

## 2. Site Measurement (现场测量)

### 2.1 Measurement Tools Required

| Tool | Purpose | Accuracy Required |
| --- | --- | --- |
| Laser distance meter | Room dimensions, ceiling height | ±1 mm |
| Steel tape measure (5 m) | Cabinet-level dimensions | ±1 mm |
| Digital level / inclinometer | Wall plumb, floor level | ±0.5 degrees |
| 2 m straightedge | Wall flatness check | Visual + gap measurement |
| Square (90 degrees) | Corner angles | Visual |
| Digital camera | Document site conditions, obstacles | N/A |
| Graph paper / CAD software | Record measurements | N/A |

### 2.2 Measurement Checklist

- Room length, width, and ceiling height (measure at multiple points)

- Wall flatness: check each wall at top, middle, and bottom

- Wall plumb: check each wall for out-of-plumb condition

- Corner angles: verify 90-degree corners (many are not exactly 90)

- Floor levelness: check across the entire cabinet run

- Location of electrical outlets, switches, and data ports

- Location of water supply lines, drain pipes, and gas lines

- HVAC vents, radiators, and heating pipes

- Window and door positions, including swing direction

- Ceiling features (beams, soffits, dropped ceilings)

- Baseboard height and profile (if cabinets sit on floor)

- Ceiling crown molding or trim (if cabinets reach ceiling)

### 2.3 Critical Tolerance Values

| Measurement | Maximum Tolerance |
| --- | --- |
| Wall flatness | 3 mm over 2 m |
| Wall plumb | 3 mm over 3 m height |
| Floor levelness | 3 mm over 2 m |
| Corner squareness | 90 ± 1 degrees |
| Ceiling height variation | 5 mm across room (note for scribe) |

### 2.4 Measurement Documentation

For each cabinet location, document: 1. Plan view sketch with dimensions 2. Elevation view showing height, shelf positions, and features 3. Photo documentation of the area from multiple angles 4. Obstacle log listing all pipes, outlets, and obstructions with dimensions 5. Adjacent furniture/fixtures that may affect installation

## 3. Production Lead Time

### 3.1 Typical Production Timeline

| Phase | Duration | Notes |
| --- | --- | --- |
| Design finalization | 3-7 days | Includes 3D renderings, material selection, revisions |
| Factory production | 15-30 days | Panel cutting, edge-banding, drilling, assembly |
| Quality inspection | 1-2 days | Factory QC check before shipping |
| Delivery scheduling | 3-7 days | Coordinate with installation date |
| Total lead time | 22-46 days | Plan accordingly in renovation schedule |

### 3.2 Factors Affecting Lead Time

| Factor | Impact | Mitigation |
| --- | --- | --- |
| Custom dimensions | +3-5 days | Measure early in renovation |
| Special materials (imported veneer, specialty glass) | +7-15 days | Select from stocked materials |
| Complex hardware (soft-close, pull-outs, LED lighting) | +3-7 days | Confirm availability during design |
| Color matching across multiple pieces | +3-5 days | Order all related cabinets together |
| Rush order | Available at 20-50% surcharge | Plan ahead to avoid rush fees |

### 3.3 Pre-Delivery Site Preparation

| Task | Timing | Responsibility |
| --- | --- | --- |
| Flooring installed and cured | 3-5 days before delivery | Flooring contractor |
| Walls painted (at least primer coat) | Before delivery | Painter |
| Ceiling finished | Before delivery | Ceiling contractor |
| Electrical rough-in complete | Before cabinet design | Electrician |
| Plumbing rough-in complete (kitchen/bath) | Before cabinet design | Plumber |
| Site clean and accessible | Day of delivery | General contractor / homeowner |

## 4. Cabinet Components

### 4.1 Common Materials

| Component | Material Options | Notes |
| --- | --- | --- |
| Cabinet body (carcass) | Particle board (18 mm), plywood (18 mm), MDF | Plywood more durable; particle board most common |
| Door/drawer fronts | MDF (painted), solid wood, laminate, veneer, acrylic | Match style and budget |
| Back panel | 5-9 mm fiberboard or plywood | Must be securely fastened for structural rigidity |
| Shelf board | 18 mm particle board or plywood | Max span 800 mm without sagging |
| Edge banding | PVC (0.5-2 mm), ABS, wood veneer | 2 mm PVC recommended for durability |
| Hardware | Hinges, slides, handles, connectors | Invest in quality hardware (Blum, Hettich, Grass) |

### 4.2 Hardware Specifications

| Hardware | Quality Indicator | Recommended |
| --- | --- | --- |
| Hinges | Soft-close, adjustable 3D, cup depth | Blum Clip Top, Hettich Sensys |
| Drawer slides | Soft-close, full extension, load rating | Blum Tandembox, Hettich InnoTech |
| Drawer runners load rating | Standard: 25 kg; Heavy-duty: 40-50 kg | Size to expected contents |
| Handles/pulls | Solid metal, secure mounting | Match cabinet style |
| Shelf pins | Metal (not plastic), at least 4 per shelf | For adjustable shelves |
| Clothes rod | Oval chrome-plated, wall-mounted brackets | Max span 900 mm |
| Pull-down wardrobe lift | Gas spring assisted, load-rated | For high wardrobe sections |
| Pantry pull-out | Full-extension slides, wire or wood basket | Coordinate with cabinet width |

## 5. Installation Process

### 5.1 Pre-Installation Checklist

- All cabinets delivered and inspected for damage

- Floor is finished, level, and clean

- Walls are painted (final coat can wait until after installation)

- All electrical and plumbing rough-ins are complete and tested

- Installation area is clear and well-lit

- Required tools are on site (see below)

- Cabinet layout plan is available and reviewed

- Installation sequence is planned (typically: base cabinets, then wall cabinets, then tall cabinets)

### 5.2 Required Tools

| Tool | Purpose |
| --- | --- |
| Laser level or water level | Establish level reference lines |
| Drill/driver (cordless) | Assembly and mounting |
| Drill bits (wood, masonry) | Pilot holes, wall anchors |
| Screwdriver set | Hardware installation |
| Circular saw or jigsaw | On-site trimming/cutting |
| Hole saw set | Pipe and wire cutouts |
| Shims (wood/plastic) | Leveling and gap filling |
| Clamps | Holding pieces during assembly |
| Rubber mallet | Adjusting fit without damage |
| Caulk gun | Gap sealing |
| Measuring tools (tape, square, level) | Verification throughout |

### 5.3 Step-by-Step Installation

#### Phase 1: Base Cabinet Installation

| Step | Action | Detail |
| --- | --- | --- |
| 1 | Mark reference lines | Use laser level to mark level line on wall at cabinet top height (typically 850-900 mm from floor for kitchen) |
| 2 | Locate wall studs | Mark stud positions; verify with stud finder |
| 3 | Position first cabinet | Place in corner or at designated start point |
| 4 | Level the cabinet | Use shims under cabinet base; check level front-to-back and side-to-side |
| 5 | Secure to wall | Drill through cabinet back rail into wall studs; use minimum 50 mm screws |
| 6 | Install adjacent cabinets | Clamp together, verify alignment, fasten with connector screws through side panels |
| 7 | Join cabinets | Use cabinet connector bolts or screws through pre-drilled holes; install cam locks if provided |
| 8 | Install toe-kick | Attach after all base cabinets are level and secured |

#### Phase 2: Wall Cabinet Installation

| Step | Action | Detail |
| --- | --- | --- |
| 1 | Mark reference lines | Bottom of wall cabinets typically 1350-1450 mm from floor (to bottom of cabinet) |
| 2 | Install hanging rail (if using rail system) | Level and secure rail to wall studs |
| 3 | Hang cabinets | Hang on rail system or mount directly to wall studs |
| 4 | Level and plumb | Adjust hanging brackets; check with level |
| 5 | Join adjacent wall cabinets | Clamp, align, and fasten with connector screws |
| 6 | Fill gaps to ceiling | Install scribe/crown molding or filler strip as needed |

#### Phase 3: Tall Cabinet Installation

| Step | Action | Detail |
| --- | --- | --- |
| 1 | Position cabinet | Place against wall in designated location |
| 2 | Level | Use shims under base; check plumb on both faces |
| 3 | Secure | Fasten to wall studs through back panel |
| 4 | Verify door clearance | Ensure adjacent doors and drawers can open fully |

### 5.4 Connection Methods

| Method | Description | Strength | Reversibility |
| --- | --- | --- | --- |
| Rail/hanging system (wall cabinets) | Cabinet hangs on wall-mounted rail | High | Yes — easy to adjust |
| Direct screw to studs | Screws through back panel into wall studs | High | No — leaves holes |
| Cabinet-to-cabinet connectors | Bolts or screws through side panels | Medium | Yes |
| L-brackets (base to wall) | Metal brackets for additional support | High | Yes |
| Adhesive (filler strips, trim) | Construction adhesive or silicone | Low-Medium | No |

## 6. Leveling and Alignment

### 6.1 Leveling Tolerances

| Item | Maximum Deviation |
| --- | --- |
| Cabinet top levelness | 1 mm over full cabinet run |
| Cabinet face plumb | 1 mm over full height |
| Door alignment (adjacent) | 1 mm gap consistency |
| Drawer front alignment | 1 mm with adjacent doors/drawers |
| Overall run alignment | 2 mm over 3 m |

### 6.2 Shim Placement Guide

| Location | Purpose |
| --- | --- |
| Under cabinet base (front) | Correct front-to-back level |
| Under cabinet base (sides) | Correct side-to-side level |
| Between cabinet and wall (back) | Correct for out-of-plumb walls |
| Between cabinets (top/bottom) | Correct for height mismatches between units |

Important: Shims must be placed directly under structural points (corners, divider panels), not in the middle of unsupported panels.

## 7. Door and Drawer Adjustment

### 7.1 Hinge Adjustment (3-Way)

| Adjustment | Screw Location | Effect |
| --- | --- | --- |
| Depth (in/out) | Rear screw on hinge arm | Moves door closer to or farther from cabinet |
| Height (up/down) | Mounting plate screws | Moves door up or down |
| Side (left/right) | Front screw on hinge arm | Moves door left or right; closes or opens gap |

### 7.2 Door Gap Specifications

| Gap Type | Recommended Size |
| --- | --- |
| Between adjacent doors | 2-3 mm |
| Between door and cabinet frame (side) | 2 mm |
| Between door and cabinet frame (top/bottom) | 2-3 mm |
| Overlay door to edge of cabinet | Per door design (full, half, or inset overlay) |

### 7.3 Adjustment Procedure

- Loosen the adjustment screws slightly (do not remove).

- Adjust one screw at a time; close door to check alignment.

- Tighten all screws once alignment is correct.

- Check operation: door should open and close smoothly without binding.

- Verify gap consistency across all doors in the same run.

### 7.4 Drawer Alignment

| Issue | Adjustment |
| --- | --- |
| Drawer front not level | Adjust slide mounting position or front panel mounting screws |
| Drawer not closing smoothly | Check slide alignment; lubricate; verify no obstructions |
| Drawer front offset from adjacent door | Adjust front panel on drawer box mounting brackets |
| Soft-close not engaging | Check slide installation; verify drawer is fully seated on slides |

## 8. Hardware Installation

### 8.1 Hardware Installation Sequence

| Order | Hardware | Notes |
| --- | --- | --- |
| 1 | Hinges | Mount to door first, then to cabinet |
| 2 | Drawer slides | Mount to cabinet first, then to drawer |
| 3 | Drawer fronts | Attach to drawer box after slides are installed |
| 4 | Handles/pulls | Mark, drill, and mount after all doors are aligned |
| 5 | Internal accessories | Shelf pins, clothes rods, pull-outs, organizers |
| 6 | Soft-close dampers | Add to cabinets without soft-close hinges |
| 7 | Lighting (LED strips, puck lights) | Wire and mount in pre-planned locations |
| 8 | Lock mechanisms | Install on designated doors/drawers |

### 8.2 Handle/Pull Placement

| Cabinet Type | Placement | Distance from Edge |
| --- | --- | --- |
| Base cabinet door | Corner, opposite hinge side | 50-60 mm from corner |
| Wall cabinet door | Corner, opposite hinge side | 50-60 mm from corner |
| Drawer | Centered horizontally | 80-120 mm from top edge |
| Tall cabinet door | Vertical edge | 50-60 mm from edge, 100-150 mm from top |
| Pull handle (appliance panel) | Per appliance spec | Per manufacturer |

### 8.3 Drilling for Handles

| Step | Action |
| --- | --- |
| 1 | Mark hole positions using template or measuring from edges |
| 2 | Verify both sides (front and back) of drilling location |
| 3 | Use appropriate drill bit for handle screw diameter |
| 4 | Drill from the finished face side to prevent tear-out |
| 5 | Apply masking tape at drilling location for clean entry |
| 6 | Deburr hole edges |
| 7 | Install handle with screws (do not overtighten) |

## 9. Gap Filling and Finishing

### 9.1 Gap Types and Treatments

| Gap Location | Width | Filling Method |
| --- | --- | --- |
| Cabinet top to ceiling | 0-30 mm | Crown molding, filler strip, or scribe |
| Cabinet side to wall | 0-20 mm | Scribe strip (cut to match wall contour), caulk |
| Between adjacent cabinets | 0-1 mm | Connector strip, caulk (painted cabinets) |
| Base cabinet to floor | 0-15 mm | Toe-kick board (adjustable or fixed) |
| Around pipes/conduits | Varies | Decorative escutcheon plate + caulk |
| Corner (cabinet to wall) | Varies | Caulk (color-matched) |

### 9.2 Scribe Strip Installation

A scribe strip is a narrow piece of material (typically 50-80 mm wide) used to fill the gap between a cabinet and an uneven wall.

| Step | Action |
| --- | --- |
| 1 | Cut scribe strip slightly wider than the maximum gap |
| 2 | Place against cabinet, flush with cabinet face |
| 3 | Use a compass or scribe tool to transfer wall contour onto the strip |
| 4 | Cut along the scribed line with a jigsaw |
| 5 | Test fit against the wall; trim as needed |
| 6 | Attach to cabinet with adhesive and/or finish nails |
| 7 | Caulk the seam between scribe and wall |

### 9.3 Caulking Guidelines

| Joint | Caulk Type | Color |
| --- | --- | --- |
| Cabinet to painted wall | Acrylic latex | Match wall paint |
| Cabinet to tile | Silicone (color-matched) | Match grout or tile |
| Cabinet to countertop | Silicone (food-safe for kitchen) | Match countertop or clear |
| Cabinet joints (painted) | Acrylic latex | Match cabinet color |
| Exterior gaps | Weatherproof silicone | Match exterior finish |

### 9.4 Finishing Checklist

- All gaps filled and caulked

- Caulk lines toooled smooth and uniform

- Excess caulk cleaned from surfaces

- All protective films removed from hardware and surfaces

- Cabinet interiors vacuumed and cleaned

- All doors and drawers tested for smooth operation

- Shelves installed at correct heights

- Touch up any installation scratches on cabinet surfaces

## 10. Quality Acceptance Criteria

| Item | Standard |
| --- | --- |
| Levelness | ≤ 1 mm deviation across full cabinet run |
| Plumb | ≤ 1 mm deviation over full height |
| Door gap consistency | 2-3 mm uniform between all adjacent doors |
| Door operation | Opens and closes smoothly; self-closes (if soft-close) without slamming |
| Drawer operation | Smooth full extension; soft-close engages; no binding |
| Surface quality | No visible scratches, chips, or dents under normal lighting |
| Hardware security | All handles, hinges, and slides firmly mounted; no wobble |
| Gap filling | All gaps filled; caulk lines uniform; no visible unfinished edges |
| Alignment with adjacent elements | Doors open fully; no interference with adjacent furniture or fixtures |

## 11. Common Installation Problems

| Problem | Cause | Solution |
| --- | --- | --- |
| Doors not closing flush | Hinge misalignment | Adjust 3-way hinge screws |
| Drawer sticks on slide | Slide not level or parallel | Re-mount slides; check cabinet square |
| Visible gaps at wall | Wall is not plumb | Use scribe strip cut to wall contour |
| Cabinet rocks forward/back | Floor not level | Shim under cabinet base |
| Scratches during installation | Tools or cabinets rubbing | Use protective padding; handle carefully |
| Soft-close not working | Slide not installed correctly or wrong load rating | Reinstall; verify weight capacity |
| Squeaking hinges or slides | Lack of lubrication | Apply silicone lubricant |
| Shelf sagging | Span too wide or load too heavy | Add center support or reduce span |

# 装修配色指南

## 色彩基础

### 色相环

- 暖色：红、橙、黄

- 冷色：蓝、绿、紫

- 中性色：黑、白、灰

### 配色原则

- 60-30-10原则

- 同色系搭配

- 对比色搭配

- 互补色搭配

## 常用配色方案

### 现代简约

| 主色 | 辅助色 | 点缀色 |
| --- | --- | --- |
| 白色 | 灰色 | 黑色/金属色 |

### 北欧风格

| 主色 | 辅助色 | 点缀色 |
| --- | --- | --- |
| 白色 | 原木色 | 蓝绿/明黄 |

### 轻奢风格

| 主色 | 辅助色 | 点缀色 |
| --- | --- | --- |
| 灰色 | 黑色 | 香槟金/墨绿 |

### 新中式

| 主色 | 辅助色 | 点缀色 |
| --- | --- | --- |
| 白色 | 胡桃木色 | 中国红 |

## 各空间配色建议

| 空间 | 推荐色系 | 避免色系 |
| --- | --- | --- |
| 客厅 | 暖色、中性色 | 过于鲜艳 |
| 卧室 | 暖色、柔和色 | 冷色 |
| 书房 | 冷色、中性色 | 过于鲜艳 |
| 厨房 | 浅色系 | 深色 |
| 卫生间 | 浅色系 | 深色 |

# 餐厅装修指南

## 布局设计

### 餐桌尺寸

| 人数 | 餐桌尺寸 |
| --- | --- |
| 2-4人 | 120×80cm |
| 4-6人 | 150×90cm |
| 6-8人 | 180×100cm |

### 通道宽度

- 餐桌周围通道≥90cm

- 餐椅拉开后通道≥60cm

## 灯光设计

- 吊灯：距桌面60-80cm

- 色温：2700-3000K

- 照度：150-200lux

## 色彩搭配

- 主色：暖色系

- 辅助色：原木色

- 点缀色：红色或橙色

## 材料选择

- 地面：瓷砖（易清洁）

- 墙面：乳胶漆

- 餐桌：实木或大理石

## 收纳设计

- 餐边柜：收纳餐具

- 酒柜：收纳酒类

- 储物柜：收纳杂物

# 排水系统施工工艺 (Drainage System Construction Craft)

## 概述

排水系统是装修中最重要的隐蔽工程之一。排水不畅、返味、渗漏等问题一旦发生，维修成本极高。本指南涵盖排水管道施工的各项技术要求，帮助施工人员和业主把控质量。

## 管道坡度要求

### 排水管坡度标准

管道坡度是保证排水顺畅的核心参数。坡度过小导致排水不畅、污物沉积；坡度过大则可能造成水封破坏。

| 管道类型 | 管径（mm） | 标准坡度 | 最小坡度 | 最大坡度 |
| --- | --- | --- | --- | --- |
| 污水管 | 50 | 0.035 | 0.025 | 0.05 |
| 污水管 | 75 | 0.025 | 0.015 | 0.035 |
| 污水管 | 110 | 0.020 | 0.012 | 0.030 |
| 污水管 | 160 | 0.010 | 0.007 | 0.015 |
| 废水管 | 50 | 0.035 | 0.025 | 0.05 |
| 废水管 | 75 | 0.025 | 0.015 | 0.035 |
| 雨水管 | 75 | 0.015 | 0.010 | 0.020 |
| 雨水管 | 110 | 0.010 | 0.007 | 0.015 |

### 坡度计算方法

坡度 = 落差 / 水平距离

例如：
  50mm 排水管，水平距离 3m
  标准坡度 0.035
  所需落差 = 3m x 0.035 = 105mm

### 坡度现场检测

| 方法 | 工具 | 操作步骤 |
| --- | --- | --- |
| 水平管法 | 透明软管 + 水 | 软管注水，两端液面高差即为落差 |
| 激光水平仪法 | 激光水平仪 + 标尺 | 测量管道两端标高差 |
| 坡度尺法 | 专用坡度尺 | 直接放置在管道上读数 |

## 通风管（通气管）设置

### 通气管作用

- 平衡管道内气压，防止水封被破坏

- 排出有害气体

- 促进排水顺畅

### 设置要求

| 项目 | 要求 |
| --- | --- |
| 通气立管 | 必须伸出屋面，高出屋面不小于 300mm |
| 通气立管 | 距屋面完成面不小于 2m（人在屋面活动时） |
| 结合通气管 | 排水立管高度超过 5m 时，每层设置 |
| 自通气管 | 排水横管长度超过规定限值时设置 |
| 环形通气管 | 排水横支管连接卫生器具超过 4 个且长度超过 12m 时设置 |

### 横管最大长度与通气

| 管径（mm） | 无通气最大长度（m） | 有通气最大长度（m） |
| --- | --- | --- |
| 50 | 6 | 12 |
| 75 | 9 | 15 |
| 110 | 12 | 不限 |
| 160 | 15 | 不限 |

### 通气管连接规则

- 通气管不得与烟道、风道连接

- 通气管出口应远离门窗和通风口

- 寒冷地区通气管出屋面部分应保温

- 通气立管上不得设置存水弯

## 清扫口（检查口）设置

### 设置要求

| 位置 | 要求 |
| --- | --- |
| 排水立管 | 每层设置，距地面 1m |
| 排水横管 | 转弯处、直线段每隔 15–20m |
| 横管起点 | 必须设置 |
| 器具排水 | 每个器具附近宜设置 |

### 清扫口类型

| 类型 | 适用场景 | 特点 |
| --- | --- | --- |
| 地面清扫口 | 地面排水横管 | 带密封盖，与地面平齐 |
| 墙面清扫口 | 墙面排水管道 | 检修方便 |
| 存水弯检修口 | 存水弯下方 | 兼作清扫口 |

### 安装注意事项

- 清扫口不得被装修面层覆盖

- 清扫口周围留有不小于 400mm 的检修空间

- 清扫口盖板应便于拆卸且密封良好

- 装饰阶段做好清扫口的成品保护

## 地漏设置

### 地漏选型

| 类型 | 原理 | 优点 | 缺点 | 适用场景 |
| --- | --- | --- | --- | --- |
| 水封式 | 存水弯水封 | 防臭效果好 | 需定期补水 | 卫生间、淋浴房 |
| 重力翻板式 | 重力自动开闭 | 排水快 | 易卡异物 | 厨房、阳台 |
| 磁悬浮式 | 磁力控制 | 防臭、防虫 | 价格较高 | 所有区域 |
| 硅胶式 | 硅胶芯自动闭合 | 安装方便 | 寿命较短 | 临时使用 |

### 地漏位置要求

| 区域 | 位置要求 |
| --- | --- |
| 淋浴房 | 淋浴区最低点，距墙 50–100mm |
| 卫生间干区 | 靠近洗手台或马桶旁 |
| 厨房 | 水槽下方或地面最低点 |
| 阳台 | 靠近雨水管，地面最低点 |
| 洗衣机位 | 洗衣机排水口正下方或附近 |

### 地面坡度（坡向地漏）

| 区域 | 坡度要求 |
| --- | --- |
| 淋浴房 | 1%–2% |
| 卫生间干区 | 0.5%–1% |
| 厨房 | 0.5%–1% |
| 阳台 | 0.5%–1% |

### 地漏安装检查清单

- 地漏顶面低于完成地面 5–10mm

- 地漏四周密封严实

- 地漏水封深度不小于 50mm

- 排水测试：倒水试验，排水通畅无积水

- 地漏周边瓷砖坡向正确

## 防臭措施

### 臭气来源

- 存水弯水封干涸

- 管道接口密封不严

- 地漏防臭失效

- 马桶法兰圈老化

- 排气管倒灌

### 防臭措施清单

| 措施 | 位置 | 施工要点 |
| --- | --- | --- |
| 存水弯 | 每个排水器具 | P 型或 S 型，水封深度 >= 50mm |
| 防臭地漏 | 所有地漏 | 选择水封式或磁悬浮式 |
| 马桶法兰圈 | 马桶底部 | 安装时对准排污口，密封严实 |
| 洗手盆下水 | 洗手盆下方 | 使用 S 型弯管或存水弯 |
| 洗菜盆下水 | 水槽下方 | S 型弯管 + 密封盖 |
| 管道接口 | 所有管道连接处 | 橡胶圈密封 + 密封胶 |
| 排气管防倒灌 | 排气管出口 | 安装止回阀 |

### 存水弯设置规范

| 器具 | 存水弯类型 | 水封深度（mm） |
| --- | --- | --- |
| 洗手盆 | P 型弯管 | >= 50 |
| 厨房水槽 | P 型弯管 | >= 50 |
| 浴缸 | 自带存水弯 | >= 50 |
| 淋浴地漏 | 水封式地漏 | >= 50 |
| 马桶 | 自带存水弯 | – |
| 洗衣机 | P 型弯管 + 专用接头 | >= 50 |

## 管道材料选择

### 常用管材

| 管材 | 适用场景 | 优点 | 缺点 |
| --- | --- | --- | --- |
| PVC-U | 室内排水横管、立管 | 价格低、易施工 | 噪音大、耐热差 |
| 静音 PVC | 室内排水立管 | 噪音小 | 价格较高 |
| 铸铁管 | 排水立管、横管 | 噪音小、耐用 | 重量大、施工难 |
| HDPE | 埋地排水管 | 耐腐蚀、柔性好 | 价格较高 |
| 不锈钢 | 明装排水管 | 美观、耐用 | 价格最高 |

### 管材规格对照

| 器具 | 推荐管径（mm） | 最小管径（mm） |
| --- | --- | --- |
| 洗手盆 | 50 | 40 |
| 厨房水槽 | 50 | 40 |
| 浴缸 | 50 | 50 |
| 淋浴 | 50 | 50 |
| 马桶 | 110 | 110 |
| 洗衣机 | 50 | 40 |
| 地漏 | 50/75 | 50 |

## 施工流程

### 标准施工步骤

- 定位放线：按图纸确定管道走向和器具位置

- 预埋/开槽：墙地面开槽或预埋套管

- 管道预制：按尺寸下料、切割、组装

- 管道安装：按坡度要求安装固定管道

- 器具连接：连接排水器具

- 闭水试验：对所有排水管道进行灌水试验

- 隐蔽验收：隐蔽前拍照记录并验收

- 封槽/回填：验收合格后封槽

### 闭水（灌水）试验

| 项目 | 要求 |
| --- | --- |
| 试验方法 | 向管道内注水至满 |
| 观察时间 | 不少于 30 分钟 |
| 合格标准 | 水位无下降、接口无渗漏 |
| 试验范围 | 所有排水管道 |
| 记录要求 | 拍照记录，各方签字 |

## 常见问题及处理

| 问题 | 可能原因 | 处理方法 |
| --- | --- | --- |
| 排水不畅 | 坡度不足、管径偏小 | 重新调整坡度或更换管径 |
| 返味 | 水封干涸、密封不严 | 补水或更换防臭地漏 |
| 渗漏 | 接口密封不良 | 重新密封接口 |
| 堵塞 | 管内有建筑垃圾 | 通管清理，施工时做好管口封堵 |
| 噪音大 | 管道固定不牢、管材选型不当 | 增加管卡、更换静音管 |
| 地漏积水 | 坡度不够、地漏位置偏高 | 调整坡度或重新安装地漏 |

## 成品保护

- 施工期间所有管口必须临时封堵

- 严禁向排水管倾倒建筑垃圾

- 严禁踩踏已安装的排水管道

- 地漏安装后加盖临时保护盖

- 管道固定后不得作为脚手架支撑

# 配电箱施工工艺 (Electrical Panel Installation Craft)

## 概述

配电箱是住宅电气系统的中枢，承担着电能分配、过载保护、漏电保护等关键功能。施工质量直接关系到用电安全，必须严格按照国家标准 GB 50303《建筑电气工程施工质量验收规范》执行。

## 一、配电箱选型

### 1.1 规格选择

| 户型面积 | 回路数量 | 箱体尺寸（宽x高x深） | 建议总开关容量 |
| --- | --- | --- | --- |
| ≤60㎡ | 6-8回路 | 280×450×120mm | C32 |
| 60-100㎡ | 10-12回路 | 360×550×150mm | C40 |
| 100-140㎡ | 14-16回路 | 420×600×180mm | C50 |
| 140-200㎡ | 18-22回路 | 500×650×200mm | C63 |
| >200㎡ | 24回路以上 | 定制 | C63或更大 |

### 1.2 断路器选型标准

| 用途 | 断路器类型 | 额定电流 | 漏电动作电流 |
| --- | --- | --- | --- |
| 总开关 | 2P空气开关 | C40-C63 | 不带漏保 |
| 照明回路 | 1P空气开关 | C16 | 无需漏保 |
| 普通插座 | 1P+N漏保 | C20 | 30mA |
| 厨房插座 | 1P+N漏保 | C25 | 30mA |
| 卫生间插座 | 1P+N漏保 | C25 | 30mA/10mA |
| 空调回路 | 1P空气开关 | C20-C25 | 无需漏保 |
| 电热水器 | 2P漏保 | C25-C32 | 10mA |
| 总漏保（如有） | 2P漏保 | C63 | 300mA（延时型） |

### 1.3 常见品牌推荐

- 一线品牌：施耐德(Schneider)、ABB、西门子(Siemens)、罗格朗(Legrand)

- 国产优质：正泰(CHINT)、德力西(DELIXI)、人民电器

- 注意：务必选择具有3C认证的产品，拒绝无品牌或仿冒产品

## 二、电路回路设计

### 2.1 标准住宅回路划分

┌──────────────────────────────────────────┐
│               总开关(C40/C63)              │
├──────────┬────┬─────┬──────┬──────┬──────┤
│ 照明回路 │ 厨房│ 卫  │客厅卧│ 空调 │ 专线 │
│ C16×2-3 │ C25 │ C25 │C20×2 │ C20  │ C32 │
└──────────┴────┴─────┴──────┴──────┴──────┘

### 2.2 回路设计原则

- 照明与插座分离：禁止共用回路

- 大功率电器独立回路：空调、电热水器、烤箱等单独走线

- 厨房卫生间独立回路：潮湿环境必须带漏保

- 空调回路可免漏保：挂机/柜机使用1P空开，中央空调需单独设计

- 冰箱回路建议独立：便于外出时单独保留冰箱供电

- 弱电回路：网络、安防、智能家居单独考虑

### 2.3 导线截面选择

| 回路类型 | 导线规格（铜芯） | 对应断路器 |
| --- | --- | --- |
| 照明 | BV 2.5mm² | C16 |
| 普通插座 | BV 2.5mm² | C20 |
| 厨房/卫生间 | BV 4mm² | C25 |
| 空调(1.5P以下) | BV 2.5mm² | C20 |
| 空调(2P以上) | BV 4mm² | C25 |
| 电热水器/烤箱 | BV 4mm² | C25-C32 |
| 进户线 | BV 10mm² | C63 |

## 三、施工工艺

### 3.1 安装位置要求

- 距地高度：1.6m-1.8m（箱体底部距地面）

- 距门边：150mm-300mm

- 避开燃气管道：水平间距≥300mm

- 避开潮湿区域：不得安装在卫生间、厨房内（公共区域除外）

- 暗装箱体：墙面开槽深度 = 箱体深度 + 20mm

- 预留检修空间：前方操作空间≥600mm

### 3.2 箱体安装步骤

- 定位放线：根据图纸确定安装位置，用水平仪标记

- 开槽打孔：使用开槽机按标记切割墙面，深度一致

- 固定箱体：使用膨胀螺栓固定，确保箱体水平（偏差≤2mm）

- 封堵边缘：箱体与墙面缝隙用水泥砂浆填实抹平

- 接地处理：箱内PE排必须可靠接地，接地电阻≤4Ω

### 3.3 接线工艺

#### 线序排列

从左到右排列顺序：
[总开关] → [照明] → [插座] → [厨房] → [卫生间] → [空调] → [专线]

#### 接线规范

- 导线颜色统一：L(火线)-红色，N(零线)-蓝色，PE(地线)-黄绿双色

- 接线端子压接牢固，使用专用压线钳

- 同一端子最多接两根导线

- 导线弯曲半径≥导线直径的6倍

- 线头裸露长度一致（12-15mm），不得过长或过短

- 箱内导线余量150mm-200mm，便于后期检修

## 四、标识标准

### 4.1 标签要求

- 每个回路必须在断路器下方粘贴标签

- 标签内容：回路用途 + 导线规格（例：“客厅插座 BV2.5”）

- 使用专用标签纸或线号机打印，禁止手写

- 标签位置统一，排列整齐

- 总开关标签注明：“总开关”或”进线开关”

### 4.2 示例标签

┌──────┐
│总开关│
│C63 2P│
└──────┘
┌──────┐
│照明1 │
│C16 1P│
└──────┘
┌──────┐
│照明2 │
│C16 1P│
└──────┘
┌──────┐
│厨房  │
│C25漏保│
└──────┘

## 五、验收标准

### 5.1 验收项目清单

| 验收项目 | 标准要求 | 检测方法 |
| --- | --- | --- |
| 箱体安装 | 牢固、水平 | 水平尺检查 |
| 导线颜色 | 红/蓝/黄绿 | 目视检查 |
| 接线紧固 | 无松动 | 手拉检查 |
| 绝缘电阻 | ≥0.5MΩ | 兆欧表测试 |
| 漏保功能 | 动作时间≤0.1s | 漏保测试仪 |
| 接地电阻 | ≤4Ω | 接地电阻测试仪 |
| 回路标识 | 清晰、准确 | 目视检查 |
| 箱内整洁 | 无杂物、无裸线 | 目视检查 |

### 5.2 常见质量问题

- 零地混接：零线接入地线排，导致漏保频繁跳闸

- 线径不匹配：大断路器配小导线，存在过载隐患

- 未做接地：PE排未接或不接地线，丧失保护功能

- 标签缺失：后期维护困难，误操作风险

- 导线交叉凌乱：影响散热，不便检修

- 箱体未接地：箱体金属外壳未做等电位连接

## 六、注意事项

安全警示： - 施工前必须确认进线电源已断开，验电后方可作业 - 严禁带电作业 - 非持证电工不得进行配电箱安装接线 - 安装完成后必须由专业人员进行绝缘电阻和漏保测试

### 6.1 施工安全要点

- 断电操作，悬挂警示牌

- 使用绝缘工具

- 一人操作，一人监护

- 测试合格后方可送电

- 送电前通知所有相关人员

### 6.2 后期维护建议

- 每半年检查一次接线端子是否松动

- 每年测试一次漏保功能

- 定期检查箱体有无发热、异响

- 保留完整的电路图，存放在固定位置

- 新增或修改回路时，及时更新标签和图纸

# 玄关装修指南

## 功能设计

### 基本功能

- 换鞋

- 挂衣

- 收纳

### 进阶功能

- 全身镜

- 换鞋凳

- 雨伞收纳

## 鞋柜设计

| 类型 | 特点 | 适用场景 |
| --- | --- | --- |
| 落地式 | 储物空间大 | 大玄关 |
| 壁挂式 | 节省空间 | 小玄关 |
| 嵌入式 | 美观 | 有嵌入式墙体 |

### 尺寸标准

- 深度：35cm

- 层高：15-20cm

- 底部留空：15cm

## 灯光设计

- 主灯：吸顶灯

- 鞋柜灯：感应灯带

- 镜前灯：有镜子时

## 材料选择

- 地面：瓷砖（耐磨）

- 墙面：乳胶漆

- 柜体：防潮板材

## 常见布局

### 一字型

- 单侧鞋柜

- 适合狭长玄关

### L型

- 转角鞋柜

- 适合转角玄关

### 对称型

- 两侧鞋柜

- 适合宽敞玄关

# 地暖施工工艺 (Floor Heating Installation Craft)

## 概述

地暖（地面辐射供暖）是通过地板辐射散热的采暖方式，具有舒适度高、节能、不占空间等优点。本指南涵盖水地暖和电地暖两种主流工艺的施工标准、验收要点及常见问题。

## 一、水地暖 vs 电地暖

### 1.1 技术对比

| 对比维度 | 水地暖 | 电地暖（发热电缆） | 电地暖（电热膜） |
| --- | --- | --- | --- |
| 初期投资 | 较高（约120-200元/㎡） | 中等（约80-150元/㎡） | 中等（约80-130元/㎡） |
| 运行成本 | 较低（燃气/热泵） | 较高（直接电加热） | 较高（直接电加热） |
| 适合面积 | ≥60㎡ | 小面积/局部 | 小面积/局部 |
| 升温速度 | 慢（4-6小时） | 较慢（2-3小时） | 快（1-2小时） |
| 层高占用 | 约8-10cm | 约3-5cm | 约2-3cm |
| 使用寿命 | 50年（管道）/15年（锅炉） | 50年 | 10-15年 |
| 维护需求 | 定期清洗管道 | 基本免维护 | 基本免维护 |
| 可分区控制 | 是 | 是 | 是 |
| 适用地板类型 | 瓷砖/石材/复合地板 | 所有类型 | 不适用于实木 |

### 1.2 选择建议

- 全屋采暖（≥80㎡）：首选水地暖，运行成本低

- 小面积/局部采暖（<40㎡）：电地暖更经济

- 有燃气/热泵条件：水地暖

- 层高受限（<2.7m）：电地暖（发热电缆）

- 卫生间局部采暖：电地暖或电毛巾架

## 二、水地暖施工工艺

### 2.1 系统组成

┌─────────┐    ┌─────────┐    ┌─────────┐    ┌─────────┐
│  热源   │───▶│  分水器  │───▶│ 地暖管道 │───▶│  回填层  │
│(锅炉/热泵)│    │(集分水器)│    │(PEX/PERT)│    │(水泥砂浆) │
└─────────┘    └─────────┘    └─────────┘    └─────────┘
                    ▲                               │
                    │                               ▼
               ┌─────────┐                    ┌─────────┐
               │  温控器  │                    │ 地面饰面 │
               └─────────┘                    └─────────┘

### 2.2 构造层次（自下而上）

| 层次 | 材料 | 厚度 | 作用 |
| --- | --- | --- | --- |
| 结构层 | 混凝土楼板 | - | 承重基层 |
| 找平层 | 水泥砂浆 | 20mm | 平整基层 |
| 防潮层 | PE薄膜 | 0.2mm | 防止潮气上升 |
| 保温层 | XPS挤塑板 | 20-30mm | 减少热量向下散失 |
| 反射层 | 铝箔反射膜 | 0.05mm | 向上反射热量 |
| 固定层 | 钢丝网/卡钉 | - | 固定管道 |
| 加热层 | PEX/PERT管 | φ20/φ16 | 热水循环散热 |
| 填充层 | 豆石混凝土 | 40-50mm | 蓄热和均热 |
| 饰面层 | 瓷砖/地板 | 8-15mm | 最终表面 |

### 2.3 管道铺设标准

#### 间距与布局

| 区域 | 管道间距 | 铺设方式 |
| --- | --- | --- |
| 一般区域 | 200mm | 回字形/螺旋形 |
| 靠外墙区域 | 150mm（加密） | 加密回字形 |
| 窗下区域 | 150mm（加密） | 加密回字形 |
| 卫生间 | 150-175mm | 回字形（避开马桶） |
| 家具下方 | 300mm（稀疏）或不铺 | 减少热损失 |

#### 管道参数

- 管径：常用φ20mm（外径），壁厚2.0mm

- 材质：PEX-A（交联聚乙烯）或PERT（耐热聚乙烯）

- 最大回路长度：≤120m（φ20管）/ ≤90m（φ16管）

- 供水温度：35-45℃（水温），表面温度25-29℃

- 供回水温差：≤10℃

### 2.4 施工步骤

- 基层处理

  - 清理基层，确保平整、无杂物

  - 检查防水（卫生间）是否合格

  - 找平层平整度偏差≤3mm/2m

- 铺设保温层

  - XPS板错缝铺设，接缝处用铝箔胶带密封

  - 墙面边界处安装边界保温条（高100mm）

  - 保温板密度≥30kg/m³，抗压强度≥150kPa

- 铺设反射膜

  - 满铺，搭接宽度≥50mm

  - 使用胶带固定，平整无褶皱

- 铺设钢丝网

  - 规格：φ2mm，网格100×100mm

  - 搭接宽度≥1格，绑扎牢固

- 管道铺设

  - 按图纸间距铺设，卡钉固定间距≤500mm

  - 弯曲半径≥5倍管径（φ20管≥100mm）

  - 管道不得有折痕、死弯

  - 穿越伸缩缝处加套管保护

  - 整根管道，中间不得有接头

- 安装分水器

  - 位置：距地300-500mm，便于操作

  - 每个回路独立阀门控制

  - 集水器在下，分水器在上

  - 连接处使用专用接头，密封严密

- 打压测试（详见第四部分）

- 回填层施工

  - 豆石混凝土比例：水泥:砂:豆石 = 1:2:3

  - 厚度：管道上方覆盖层≥30mm，总厚度≥50mm

  - 分两次浇筑：第一次覆盖管道，第二次找平

  - 养护期：≥7天，期间禁止踩踏

  - 设置伸缩缝：面积>30㎡或边长>6m时设置

- 养护与干燥

  - 回填层养护≥7天

  - 首次升温：水温从20℃起，每天升温5℃，至设定温度

  - 地面完全干燥后方可铺设地板（通常需28天）

## 三、电地暖施工工艺

### 3.1 发热电缆施工要点

- 基层处理：平整、干燥、无尖锐物

- 铺设保温层：XPS板或保温砂浆

- 铺设反射层：铝箔反射膜

- 固定电缆：

  - 使用专用卡带固定，间距≤300mm

  - 严禁电缆交叉重叠

  - 电缆与墙体间距≥100mm

  - 弯曲半径≥6倍电缆直径

- 安装温控器：

  - 地面温度传感器埋设在两根电缆中间位置

  - 传感器探头距地面2-3mm

  - 温控器安装高度1.4m

- 绝缘测试：施工前后均需测试，绝缘电阻≥1000MΩ

- 回填：专用找平砂浆，厚度30-40mm

### 3.2 电热膜施工要点

- 适用于干燥区域，不得用于卫生间

- 膜与膜之间间距50-100mm

- 使用专用连接卡连接，绝缘密封处理

- 上方直接铺设地板，无需回填

## 四、打压测试

### 4.1 测试流程

| 步骤 | 操作 | 要求 |
| --- | --- | --- |
| 1 | 注水排气 | 缓慢注水，排尽空气 |
| 2 | 初步加压 | 加压至0.6MPa，保持10min |
| 3 | 检查渗漏 | 观察各接头、管道 |
| 4 | 稳压测试 | 加压至0.8MPa（工作压力的1.5倍） |
| 5 | 稳压保持 | 保持2小时，压降≤0.03MPa |
| 6 | 保压回填 | 回填过程中保持0.4-0.6MPa压力 |
| 7 | 回填后复测 | 再次加压至0.6MPa，保持24小时 |

### 4.2 验收标准

- 稳压期间压降≤0.03MPa

- 各接头、管道无渗漏

- 压力表读数稳定

- 测试全程有监理/业主在场确认

- 测试记录签字存档

## 五、地面饰面兼容性

### 5.1 材料选择

| 饰面材料 | 热传导性 | 适用性 | 注意事项 |
| --- | --- | --- | --- |
| 瓷砖/石材 | 优良 | ★★★★★ | 最佳选择，导热快 |
| 复合地板 | 良好 | ★★★★ | 需选择地暖专用，厚度≤15mm |
| 强化地板 | 良好 | ★★★★ | 需环保等级E0以上 |
| SPC石塑地板 | 良好 | ★★★★ | 耐热性好 |
| 实木地板 | 差 | ★★ | 易变形开裂，不推荐 |
| 实木复合地板 | 中等 | ★★★ | 可用，但导热效率低 |
| 地毯 | 极差 | ★ | 严重影响散热，不推荐 |

### 5.2 地板安装注意事项

- 地暖开启前48小时开始逐步升温

- 地面表面温度不超过29℃

- 地板铺设前地面含水率≤1.5%（水泥基层）

- 使用地暖专用地板胶或锁扣地板

- 大面积铺设时预留伸缩缝

## 六、常见问题与处理

### 6.1 施工质量通病

| 问题 | 原因 | 处理措施 |
| --- | --- | --- |
| 局部不热 | 管道间距过大/堵塞 | 重新检查管道布局 |
| 温度不均 | 回路长度差异大 | 调整分水器流量 |
| 地面开裂 | 未设伸缩缝/养护不足 | 重新设置伸缩缝 |
| 漏水 | 管道损伤/接头松动 | 局部维修或更换 |
| 升温缓慢 | 保温层不足/回填过厚 | 加强保温，优化回填 |
| 能耗过高 | 温控器设置不当 | 优化控制策略 |

### 6.2 节能使用建议

- 无人时将温度调至15-18℃（防冻模式）

- 使用温控器编程控制，分时段运行

- 定期清洗管道（每2-3年一次）

- 检查保温层完整性

- 合理设置各房间温度（卧室18-20℃，客厅20-22℃）

# 地面找平施工工艺 (Floor Leveling Construction Craft)

## 概述

地面找平是装修中承上启下的关键工序，直接影响后续地板铺装、瓷砖铺贴的质量和使用寿命。找平不合格会导致地板起翘、瓷砖空鼓开裂、门扇无法正常开启等一系列问题。

## 找平方法对比

### 水泥砂浆找平 vs 自流平

| 对比项目 | 水泥砂浆找平 | 自流平（水泥基） |
| --- | --- | --- |
| 适用厚度 | 20–50mm | 2–10mm |
| 平整度 | 3mm/2m | 2mm/2m |
| 施工难度 | 较高（需手工找平） | 较低（自动流平） |
| 干燥时间 | 28 天（完全干燥） | 24–48 小时（可行走） |
| 强度 | C20–C25 | C25–C30 |
| 成本 | 40–80 元/m2 | 60–150 元/m2 |
| 适用场景 | 大面积、落差大 | 小落差、高精度要求 |
| 对基层要求 | 较宽松 | 较严格 |

### 选择建议

| 情况 | 推荐方案 |
| --- | --- |
| 地面落差 > 20mm | 水泥砂浆找平 |
| 地面落差 5–20mm | 水泥砂浆找平（厚层） |
| 地面落差 < 5mm | 自流平 |
| 铺实木地板 | 水泥砂浆找平（需要打龙骨基层） |
| 铺复合地板 | 自流平（要求高平整度） |
| 铺 PVC/橡胶地板 | 自流平（要求最高平整度） |
| 铺地暖 | 水泥砂浆找平（回填兼找平） |

## 水泥砂浆找平施工

### 材料要求

| 材料 | 规格要求 |
| --- | --- |
| 水泥 | P.O 32.5 或 P.O 42.5 硅酸盐水泥 |
| 砂 | 中砂，含泥量不超过 3%，过 5mm 筛 |
| 水 | 饮用水标准 |
| 配合比 | 水泥:砂 = 1:2.5–1:3（体积比） |
| 水灰比 | 0.4–0.5 |

### 施工步骤

#### 第一步：基层处理

- 清理基层：铲除浮浆、油污、松散层

- 修补坑洞：用水泥砂浆填补深度超过 10mm 的坑洞

- 湿润基层：施工前 24 小时充分湿润，施工前无明水

#### 第二步：标高控制

- 在墙面弹出水平基准线（通常高于地面 50cm）

- 根据基准线在地面做出灰饼（间距 1.5m）

- 灰饼顶面即为找平层完成面标高

灰饼布置示意：

┌─────────────────────┐
│  ·     ·     ·     · │
│                     │
│  ·     ·     ·     · │
│                     │
│  ·     ·     ·     · │
└─────────────────────┘

#### 第三步：铺设砂浆

- 在基层上涂刷素水泥浆一遍（水灰比 0.4–0.5）

- 随刷随铺砂浆，间隔不超过 30 分钟

- 用刮杠以灰饼为准刮平

- 用木抹子搓平

#### 第四步：压光

- 第一遍压光：砂浆初凝前（约 2–3 小时），用铁抹子压光

- 第二遍压光：表面收水后（约 4–5 小时），用铁抹子再次压光

- 第三遍压光：终凝前（约 6–8 小时），消除抹痕

#### 第五步：养护

- 终凝后（约 12 小时）开始养护

- 覆盖薄膜或湿草帘，保持湿润

- 养护期不少于 7 天

- 每天洒水 2–3 次

### 厚度要求

| 区域 | 最小厚度 | 说明 |
| --- | --- | --- |
| 一般房间 | 20mm | 过薄易开裂 |
| 厨房/卫生间 | 20mm | 需考虑坡度 |
| 有地暖区域 | 回填层 30mm + 找平层 20mm | 地暖回填一体找平 |
| 阳台 | 20mm | 注意向外排水坡度 |

## 自流平施工

### 材料要求

| 材料 | 要求 |
| --- | --- |
| 自流平水泥 | 符合 JC/T 1023 标准 |
| 界面剂 | 水性或溶剂型，与自流平兼容 |
| 用水量 | 严格按照产品说明书 |
| 搅拌设备 | 专用搅拌机，转速 500–800rpm |

### 施工步骤

#### 第一步：基层处理

- 基层强度不低于 C20

- 基层含水率不超过 8%

- 打磨去除浮浆和松散层

- 用吸尘器彻底清洁

#### 第二步：涂刷界面剂

- 界面剂均匀涂刷 1–2 遍

- 第二遍在第一遍干燥后进行

- 涂刷后等待 4–24 小时（按产品说明）

#### 第三步：搅拌自流平材料

- 按比例加水，使用专用搅拌机搅拌

- 搅拌时间 2–3 分钟

- 搅拌后静置 1 分钟消泡

#### 第四步：浇筑

- 将搅拌好的材料倒入地面

- 用齿形刮板辅助摊铺

- 用消泡滚筒排除气泡

- 材料自然流平

#### 第五步：养护

- 施工后 24 小时内避免振动

- 关闭门窗，避免强风直吹

- 24 小时后可轻度行走

- 48–72 小时后可进行后续施工

### 环境要求

| 参数 | 要求 |
| --- | --- |
| 施工温度 | 5–35 度 C |
| 基层温度 | 不低于 5 度 C |
| 空气湿度 | 不超过 75% |
| 通风 | 施工期间关闭，养护期间可微通风 |

## 平整度检测

### 检测方法

| 方法 | 工具 | 操作步骤 |
| --- | --- | --- |
| 靠尺法 | 2m 靠尺 + 塞尺 | 靠尺贴地，塞尺测最大间隙 |
| 激光法 | 激光水平仪 | 投射水平线，测量各点偏差 |
| 水准仪法 | 水准仪 + 标尺 | 各点标高测量，计算偏差 |

### 容差标准

| 后续面层 | 平整度要求 | 检测方法 |
| --- | --- | --- |
| 瓷砖 | 3mm/2m | 靠尺法 |
| 实木地板 | 3mm/2m | 靠尺法 |
| 复合地板 | 2mm/2m | 靠尺法 |
| PVC 地板 | 2mm/2m | 靠尺法 |
| 地毯 | 4mm/2m | 靠尺法 |

### 常见问题及处理

| 问题 | 原因 | 处理方法 |
| --- | --- | --- |
| 表面起砂 | 水灰比过大、水泥用量不足 | 涂刷界面剂后重新做面层 |
| 裂缝 | 基层未处理、养护不足 | 宽度 < 0.3mm 可忽略，> 0.3mm 需修补 |
| 空鼓 | 基层未湿润、素水泥浆未随刷随铺 | 空鼓面积超过 400cm2 需返工 |
| 起壳 | 压光时间不当 | 严重起壳需铲除重做 |
| 高低差 | 灰饼标高不准确 | 打磨高点，补低处 |

## 干燥时间参考

### 后续施工等待时间

| 找平方法 | 可踩踏时间 | 可铺地板时间 | 可贴瓷砖时间 | 完全干燥时间 |
| --- | --- | --- | --- | --- |
| 水泥砂浆（20mm） | 3–5 天 | 14–21 天 | 7–14 天 | 28 天 |
| 水泥砂浆（30mm） | 5–7 天 | 21–28 天 | 14–21 天 | 28 天 |
| 水泥砂浆（50mm） | 7–10 天 | 28–35 天 | 21–28 天 | 28 天以上 |
| 自流平（3mm） | 24 小时 | 3–7 天 | 3–5 天 | 7 天 |
| 自流平（5mm） | 48 小时 | 5–7 天 | 5–7 天 | 7 天 |

### 含水率检测方法

- 塑料薄膜法：1m x 1m 塑料薄膜四边密封于地面，24 小时后检查薄膜内侧有无水珠

- 湿度仪法：使用专业湿度检测仪，含水率低于 8% 方可施工

- 经验法：地面颜色均匀一致、无深色湿润斑块

## 施工检查清单

### 施工前

- 基层清理干净，无浮浆、油污

- 基层含水率符合要求

- 水平基准线已弹出

- 灰饼已做好

- 材料已进场并检验合格

- 环境温度符合施工要求

### 施工中

- 配合比准确

- 素水泥浆随刷随铺

- 厚度符合要求

- 压光遍数和时间符合要求

- 表面无裂缝、起砂

### 施工后

- 养护期足够

- 平整度检测合格

- 含水率检测合格

- 无空鼓、裂缝等缺陷

- 标高偏差在允许范围内

# 新风系统指南

## 新风系统作用

- 换气：排出室内污浊空气

- 过滤：过滤PM2.5

- 热交换：节能

## 类型对比

| 类型 | 特点 | 价格 |
| --- | --- | --- |
| 中央新风 | 全屋换气 | 5000-15000 |
| 壁挂新风 | 单房间 | 2000-6000 |
| 窗式新风 | 安装简单 | 1000-3000 |

## 选型要点

### 风量

风量 = 面积 × 层高 × 换气次数

一般住宅换气次数0.5-1次/小时

### 过滤

- 初效过滤：大颗粒

- HEPA过滤：PM2.5

- 活性炭：除甲醛

### 热交换

- 全热交换：温度+湿度

- 显热交换：只交换温度

## 安装要点

- 管道走顶

- 进排风口距离≥1m

- 主机远离卧室

- 预留检修口

## 维护

- 初效滤芯每月清洗

- HEPA滤芯每半年更换

- 活性炭滤芯每年更换

# 地暖施工详解

## 地暖类型

| 类型 | 特点 | 价格 |
| --- | --- | --- |
| 水地暖 | 舒适、寿命长 | 100-200元/㎡ |
| 电地暖 | 安装快 | 80-150元/㎡ |
| 石墨烯地暖 | 薄、节能 | 150-300元/㎡ |

## 水地暖施工

### 施工流程

- 清理基层

- 铺设保温板

- 铺设反射膜

- 铺设钢丝网

- 盘管

- 安装分水器

- 打压测试

- 回填

### 关键指标

| 项目 | 标准 |
| --- | --- |
| 管间距 | 15-20cm |
| 回填厚度 | 3-5cm |
| 打压测试 | 0.6MPa |

## 电地暖施工

### 施工流程

- 清理基层

- 铺设保温板

- 铺设发热电缆

- 安装温控器

- 测试

- 回填

## 注意事项

- 层高降低5-8cm

- 地面材料选地砖最佳

- 回填后才能铺地面

- 每个房间单独回路

# 暖通与中央空调指南

## 采暖方式选择

| 方式 | 初装费 | 使用费 | 舒适度 | 适用场景 |
| --- | --- | --- | --- | --- |
| 地暖 | 中高 | 中 | 高 | 全屋 |
| 暖气片 | 中 | 中 | 中 | 局部 |
| 空调 | 中 | 高 | 中 | 全屋 |
| 中央空调 | 高 | 中高 | 高 | 全屋 |

## 地暖

### 类型

- 水地暖：锅炉加热热水，舒适度高

- 电地暖：发热电缆或电热膜

### 施工要点

- 保温板铺设

- 反射膜铺设

- 盘管或电缆

- 回填

- 铺地面材料

### 注意事项

- 层高降低 5-8cm

- 施工周期长

- 地面材料选地砖最佳

## 中央空调

### 类型

- 一拖多：一台外机带多台内机

- 风管机：隐藏在吊顶内

### 选型要点

- 制冷量：150-200W/㎡

- 能效比：越高越省电

- 噪音：内机≤25dB

### 安装时机

- 水电阶段同步施工

- 吊顶前完成管道铺设

- 油漆阶段安装内机

## 新风系统

### 作用

- 换气

- 过滤 PM2.5

- 热交换（节能）

### 选型要点

- 风量：30-50m³/h/人

- 过滤效率：HEPA H13 以上

- 热交换率≥70%

### 安装要点

- 管道走顶

- 进排风口距离≥1m

- 主机位置远离卧室

# 儿童房装修指南

## 设计原则

### 安全

- 材料环保（ENF级）

- 避免尖锐边角

- 插座带保护盖

### 成长

- 家具可调节高度

- 色彩不宜过于鲜艳

- 预留成长空间

### 功能

- 学习区

- 休息区

- 游戏区

## 色彩搭配

| 年龄段 | 推荐色系 |
| --- | --- |
| 0-3岁 | 柔和浅色系 |
| 3-6岁 | 活泼亮色系 |
| 6-12岁 | 安静中性色系 |
| 12岁以上 | 简约色系 |

## 材料选择

- 地板：实木复合

- 墙面：环保乳胶漆

- 家具：圆角设计

## 灯光设计

- 主灯：吸顶灯，4000K

- 台灯：学习用，照度300-500lux

- 夜灯：柔和暖光

# 厨房装修注意事项

## 厨房布局详解

### 工作三角

冰箱（储存）→ 水槽（清洗）→ 灶台（烹饪），三者距离总和 3-6m 最佳。

### 各区域尺寸

| 区域 | 最小尺寸 |
| --- | --- |
| 操作台深度 | 60cm |
| 操作台高度 | 身高÷2+5cm |
| 通道宽度 | 90cm |
| 吊柜底部高度 | 距台面60-70cm |

## 水电预留清单

### 电路

- 冰箱插座（单独回路）

- 油烟机插座

- 灶具插座

- 微波炉/烤箱插座

- 净水器插座

- 垃圾处理器插座

- 洗碗机插座

- 水槽下插座（小厨宝/净水器）

### 水路

- 冷热水口

- 净水器进水口

- 洗碗机进水排水

- 垃圾处理器排水

## 材料选择

### 台面

| 材料 | 价格 | 优点 | 缺点 |
| --- | --- | --- | --- |
| 石英石 | 中 | 硬度高 | 接缝可见 |
| 岩板 | 高 | 抗菌 | 贵 |
| 不锈钢 | 中 | 耐用 | 易刮花 |

### 橱柜

- 柜体：多层实木板

- 门板：双饰面板或 PET

- 台面：石英石

### 地面

- 防滑瓷砖

- 规格 300×300 或 400×400

### 墙面

- 釉面砖

- 规格 300×600

- 灶台区域用大板减少缝隙

## 通风排烟

- 抽油烟机风量≥18m³/min

- 烟道安装止逆阀

- 厨房门留通风口

## 照明设计

- 主灯：集成吊顶灯

- 吊柜下方灯带（切菜区）

- 色温 4000K

# 厨房装修指南

## 厨房布局

### 常见布局

| 布局 | 适用空间 | 优点 | 缺点 |
| --- | --- | --- | --- |
| 一字型 | 狭长厨房 | 节省空间 | 操作距离长 |
| L型 | 方形厨房 | 操作方便 | 转角利用差 |
| U型 | 大厨房 | 操作空间大 | 需要较大空间 |
| 岛台型 | 开放式厨房 | 互动性好 | 需要很大空间 |

### 黄金三角

冰箱 → 水槽 → 灶台，三者形成三角形，总长在 3-6m 最佳。

## 橱柜设计

### 高度

- 操作台高度 = 身高 ÷ 2 + 5cm

- 灶台区比操作台低 5-10cm

### 尺寸

- 吊柜底部距操作台 60-70cm

- 地柜深度 60cm

- 吊柜深度 35cm

## 水电预留

### 电路

- 冰箱单独回路

- 插座带开关

- 预留净水器、垃圾处理器插座

### 水路

- 冷热水口

- 净水器进水口

- 洗碗机进水排水

## 材料选择

### 台面

| 材料 | 价格 | 优点 | 缺点 |
| --- | --- | --- | --- |
| 石英石 | 中高 | 硬度高、耐污 | 接缝可见 |
| 岩板 | 高 | 美观、抗菌 | 价格高 |
| 不锈钢 | 中 | 耐用、易清洁 | 易刮花 |

### 门板

- 双饰面板：性价比高

- PET：易清洁

- 实木：质感好

## 通风排烟

- 抽油烟机风量≥18m³/min

- 烟道止逆阀必须安装

- 厨房门留通风口

# 大户型装修指南

## 大户型定义

- 四居室：120-150㎡

- 五居室及以上：150㎡+

- 别墅：200㎡+

## 设计要点

### 功能分区

- 动静分离

- 公私分离

- 主仆分离（别墅）

### 空间规划

- 客厅可分区域设计

- 主卧套房设计

- 独立书房/影音室

- 家政间

### 风格选择

- 大空间适合新中式、法式、美式

- 可以做复杂造型

- 可以搭配多种材质

## 材料选择

- 地面：大规格瓷砖或实木地板

- 墙面：壁纸+涂料组合

- 吊顶：复杂造型吊顶

## 预算参考

| 面积 | 舒适型 | 品质型 |
| --- | --- | --- |
| 120㎡ | 15-25万 | 25-40万 |
| 150㎡ | 20-35万 | 35-55万 |
| 200㎡ | 30-50万 | 50-80万 |

## 注意事项

- 中央空调匹数要够

- 热水系统考虑多点用水

- 网络覆盖全屋

- 照明分区控制

# 照明设计指南

## 照明类型

| 类型 | 作用 | 适用场景 |
| --- | --- | --- |
| 基础照明 | 整体照亮空间 | 吸顶灯、筒灯 |
| 重点照明 | 突出特定区域 | 射灯、台灯 |
| 装饰照明 | 营造氛围 | 灯带、壁灯 |

## 色温选择

| 色温 | 光色 | 适用空间 |
| --- | --- | --- |
| 2700-3000K | 暖白 | 卧室、餐厅 |
| 4000K | 自然白 | 客厅、书房 |
| 5000-6500K | 冷白 | 厨房、卫生间 |

## 各空间照明方案

### 客厅

- 主灯：吸顶灯或吊灯

- 辅助：射灯、落地灯

- 建议：无主灯设计（筒灯+灯带）

### 卧室

- 主灯：暖色吸顶灯

- 辅助：床头灯、壁灯

- 建议：双控开关

### 厨房

- 主灯：集成吊顶灯

- 辅助：吊柜下方灯带

### 卫生间

- 主灯：集成吊顶灯

- 辅助：镜前灯

## 照度标准

| 空间 | 建议照度（lux） |
| --- | --- |
| 客厅 | 100-150 |
| 卧室 | 75-100 |
| 书房 | 300-500 |
| 厨房 | 150-200 |
| 卫生间 | 100-150 |

## 节能建议

- 使用 LED 灯具

- 分区控制

- 安装调光器

- 利用自然光

# 客厅装修指南

## 布局设计

### 沙发摆放

- 靠实墙

- 与电视距离2.5-3.5m

- 与茶几间距40-50cm

### 电视墙设计

- 简约为主

- 可做收纳柜

- 隐藏电线

## 色彩搭配

| 风格 | 主色 | 辅助色 | 点缀色 |
| --- | --- | --- | --- |
| 现代简约 | 白色 | 灰色 | 黑色 |
| 北欧 | 浅灰 | 原木色 | 明黄 |
| 轻奢 | 灰色 | 黑色 | 金色 |

## 灯光设计

- 主灯：吸顶灯或吊灯

- 辅助灯：筒灯或灯带

- 氛围灯：落地灯

## 材料选择

- 地面：瓷砖或实木复合

- 墙面：乳胶漆或壁布

- 吊顶：石膏板或不做

## 收纳设计

- 电视柜：隐藏电线和杂物

- 沙发边几：遥控器、杂志

- 角落储物柜

# 小户型装修指南

## 小户型定义

- 一居室：≤50㎡

- 小两居：50-70㎡

- 小三居：70-90㎡

## 设计原则

### 空间最大化

- 开放式布局

- 减少隔断

- 多功能空间

### 视觉放大

- 浅色系

- 镜面反射

- 通透材质

### 收纳最大化

- 定制到顶柜

- 利用垂直空间

- 隐藏式收纳

## 各空间设计

### 客厅

- 沙发选小尺寸

- 茶几选轻便型

- 电视壁挂节省空间

### 卧室

- 榻榻米+衣柜一体

- 床头柜选壁挂式

- 利用床下空间

### 厨房

- L 型或一字型布局

- 吊柜到顶

- 嵌入式电器

### 卫生间

- 干湿分离

- 壁挂马桶

- 镜柜收纳

## 材料选择

- 地板：浅色复合地板

- 墙面：浅色乳胶漆

- 家具：浅色系、多功能

## 预算参考

| 面积 | 经济型 | 舒适型 |
| --- | --- | --- |
| 50㎡ | 4-6万 | 6-10万 |
| 70㎡ | 6-9万 | 9-15万 |
| 90㎡ | 8-12万 | 12-20万 |

# 全屋智能装修

## 智能系统架构

云端/本地中枢
├── 照明系统
├── 安防系统
├── 环境系统
├── 窗帘系统
├── 影音系统
└── 传感器网络

## 装修阶段预留

### 电路预留

- 开关盒预留零线

- 窗帘盒预留插座

- 马桶旁预留插座

- 水槽下方预留插座

### 网络

- 全屋网线或mesh覆盖

- 每个房间预留网口

- 弱电箱留电源

## 智能设备清单

### 入门级

| 设备 | 价格 | 功能 |
| --- | --- | --- |
| 智能门锁 | 1000-3000 | 指纹开锁 |
| 智能开关 | 100-200/个 | 远程控制灯光 |
| 智能音箱 | 200-500 | 语音控制 |

### 进阶级

| 设备 | 价格 | 功能 |
| --- | --- | --- |
| 全屋灯光 | 3000-8000 | 场景控制 |
| 智能窗帘 | 2000-5000 | 定时开关 |
| 安防系统 | 2000-5000 | 监控报警 |

### 高级级

| 设备 | 价格 | 功能 |
| --- | --- | --- |
| 全屋智能 | 10000-30000 | 全屋联动 |
| 暖通智能 | 5000-10000 | 温控联动 |
| 影音智能 | 5000-15000 | 场景联动 |

## 主流平台

| 平台 | 特点 | 生态 |
| --- | --- | --- |
| 米家 | 性价比高 | 丰富 |
| 华为 | 通信稳定 | 扩展中 |
| Apple | 苹果生态 | 精品 |
| Aqara | 兼容性好 | 丰富 |

# 软装搭配指南

## 软装包含内容

- 家具（沙发、床、餐桌椅等）

- 窗帘布艺

- 地毯

- 灯具

- 装饰画

- 绿植

- 摆件

- 床上用品

## 软装风格匹配

### 现代简约

- 线条简洁的家具

- 几何图案装饰画

- 纯色或简单图案窗帘

### 北欧风格

- 原木色家具

- 棉麻材质窗帘

- 绿植装饰

- 温暖地毯

### 轻奢风格

- 丝绒面料家具

- 金属元素装饰

- 大理石纹理摆件

## 色彩搭配原则

### 60-30-10 原则

- 60% 主色（墙面、地板）

- 30% 辅助色（家具、窗帘）

- 10% 点缀色（抱枕、摆件）

### 常见配色方案

| 方案 | 主色 | 辅助色 | 点缀色 |
| --- | --- | --- | --- |
| 温暖 | 米白 | 驼色 | 橙红 |
| 清新 | 浅灰 | 蓝绿 | 明黄 |
| 高级 | 深灰 | 黑色 | 金色 |

## 窗帘选择

| 空间 | 材质 | 遮光度 |
| --- | --- | --- |
| 客厅 | 棉麻、纱帘 | 30-50% |
| 卧室 | 绒布、遮光布 | 70-90% |
| 书房 | 百叶窗 | 50-70% |

## 灯光搭配

- 主灯 + 辅助灯 + 氛围灯

- 色温统一或分区设置

- 智能灯光可调色调

# 隔音施工工艺 — Soundproofing Construction Craft

## 1. Overview

Effective soundproofing addresses two distinct problems: airborne sound (voices, music, TV) and impact sound (footsteps, dropped objects, furniture movement). This guide covers wall soundproofing, floor impact noise reduction, ceiling treatment, door and window sealing, and a comprehensive materials comparison for residential soundproofing projects.

## 2. Sound Fundamentals

### 2.1 Key Acoustic Metrics

| Metric | Measures | Unit | Target Value |
| --- | --- | --- | --- |
| STC (Sound Transmission Class) | Airborne sound reduction through walls/floors | Rating (higher = better) | STC 50+ for bedrooms |
| IIC (Impact Insulation Class) | Impact sound reduction through floors | Rating (higher = better) | IIC 50+ for multi-story |
| NRC (Noise Reduction Coefficient) | Sound absorption within a room | 0.0-1.0 (higher = better) | NRC 0.7+ for treatment panels |
| dB (Decibel) | Sound pressure level | dB | Reduce by 10+ dB for noticeable improvement |

### 2.2 Sound Types

| Type | Description | Examples | Transmission Path |
| --- | --- | --- | --- |
| Airborne sound | Sound traveling through air | Voices, music, TV | Through walls, doors, windows, gaps |
| Impact sound | Vibrations from physical contact | Footsteps, dragging furniture | Through floor structure to ceiling below |
| Flanking sound | Sound bypassing the primary barrier | Through ducts, pipes, structural connections | Indirect paths around insulation |

### 2.3 Four Principles of Soundproofing

| Principle | Mechanism | Application |
| --- | --- | --- |
| Mass | Heavier materials block more sound | Double-layer drywall, mass-loaded vinyl |
| Damping | Converts sound energy to heat | Green Glue, viscoelastic compounds |
| Decoupling | Separates structural connections | Resilient channels, staggered studs, floating floors |
| Absorption | Traps sound within cavity | Rock wool, fiberglass, acoustic foam |

Maximum effectiveness is achieved by combining all four principles, not relying on any single method.

## 3. Wall Soundproofing

### 3.1 Wall Construction Types

| Construction Type | STC Rating | Description | Best For |
| --- | --- | --- | --- |
| Standard 2x4 stud + single 12.5 mm drywall each side | STC 33-35 | Basic partition wall | Non-critical rooms |
| 2x4 stud + double 12.5 mm drywall each side | STC 40-43 | Added mass on standard frame | Moderate improvement |
| Staggered stud wall | STC 45-48 | Studs alternate sides, decoupling each face | Good airborne sound blocking |
| Double stud wall (two separate frames) | STC 50-55 | Two independent wall frames with gap | Best residential wall isolation |
| Resilient channel + single drywall | STC 45-50 | Metal channels decouple drywall from studs | Retrofit-friendly |
| Resilient channel + double drywall + Green Glue | STC 55-60 | Maximum mass + decoupling + damping | Home theater, music room |

### 3.2 Wall Soundproofing — Step-by-Step (New Construction)

| Step | Action | Detail |
| --- | --- | --- |
| 1 | Build staggered or double stud wall | Use 2x4 studs on alternating plates, or build two independent wall frames with 25-50 mm gap |
| 2 | Install insulation | Fill cavity with rock wool (density ≥ 60 kg/m³); cut to fit snugly around pipes and wires |
| 3 | Seal penetrations | Use acoustic sealant around all electrical boxes, pipes, and penetrations |
| 4 | First layer of drywall | Install 12.5 mm drywall; seal all perimeter joints with acoustic sealant |
| 5 | Apply damping compound (optional but recommended) | Apply Green Glue or equivalent to back of second drywall layer |
| 6 | Second layer of drywall | Stagger seams relative to first layer; offset by at least 300 mm |
| 7 | Seal all edges | Apply acoustic sealant at top plate, bottom plate, corners, and around outlets |
| 8 | Finish | Tape, mud, and paint as normal |

### 3.3 Wall Soundproofing — Retrofit (Existing Wall)

| Method | Added STC | Description | Disruption Level |
| --- | --- | --- | --- |
| Add mass-loaded vinyl (MLV) + new drywall | +5-8 | Staple MLV to existing wall, add new drywall layer | Medium |
| Add second layer of drywall with Green Glue | +8-12 | Apply Green Glue to new drywall, install over existing | Low-Medium |
| Build independent stud wall in front | +10-15 | New 50-75 mm frame, insulation, double drywall | High |
| Acoustic panels (surface mount) | +2-3 (absorption only) | Decorative panels; reduces echo, not transmission | Low |

### 3.4 Electrical Outlet Treatment

| Step | Action |
| --- | --- |
| 1 | Use putty pads or acoustic putty around outlet box perimeter |
| 2 | Install foam gasket behind outlet faceplate |
| 3 | For maximum isolation: use back-to-back outlet isolation boxes |
| 4 | Alternatively: stagger outlets so they are not directly opposite each other |

## 4. Floor Impact Noise Reduction

### 4.1 Impact Noise Sources

| Source | Typical dB Level | Transmission Path |
| --- | --- | --- |
| Normal walking | 40-50 dB | Floor structure → ceiling below |
| Hard-soled shoes | 50-65 dB | Floor structure → ceiling below |
| Dropped object | 60-80 dB | Floor structure → ceiling below |
| Furniture dragging | 55-75 dB | Floor structure → ceiling below |
| Running/jumping (children) | 60-80 dB | Floor structure → ceiling below |

### 4.2 Floor Soundproofing Methods (New Construction)

| Method | IIC Improvement | Description | Cost Level |
| --- | --- | --- | --- |
| Rubber underlayment beneath flooring | IIC +10-15 | 3-6 mm rubber mat under laminate/engineered wood | Low |
| Cork underlayment | IIC +8-12 | 3-6 mm cork sheet under flooring | Low-Medium |
| Mass-loaded vinyl under flooring | IIC +5-8 | MLV layer between subfloor and finish floor | Low |
| Floating floor system | IIC +15-20 | Decoupled floor layer on resilient pads | Medium-High |
| Concrete topping on isolation mat | IIC +20-25 | 40-50 mm concrete slab on acoustic mat | High |

### 4.3 Floor Soundproofing — Retrofit

| Method | IIC Improvement | Floor Height Added | Description |
| --- | --- | --- | --- |
| Area rugs with thick pads | IIC +5-10 | 15-25 mm | Simplest and most cost-effective |
| Acoustic underlayment + new flooring | IIC +10-15 | 10-20 mm | Remove existing floor, add underlayment, reinstall |
| Floating floor over existing floor | IIC +15-20 | 30-50 mm | Build decoupled floor on top of existing |

### 4.4 Floating Floor Construction (New Build)

| Layer (bottom to top) | Material | Thickness |
| --- | --- | --- |
| Structural subfloor | Concrete or plywood | 18-25 mm |
| Isolation membrane | Acoustic mat or rubber pad | 5-15 mm |
| Isolation strip | Perimeter foam strip (decouples from walls) | 10 mm |
| Floating subfloor | Plywood or cement board | 15-18 mm |
| Finish flooring | Laminate, engineered wood, tile | 8-15 mm |

Critical Detail: The floating floor must NOT contact any wall or vertical element. Leave a 10 mm perimeter gap filled with compressible foam.

## 5. Ceiling Soundproofing

### 5.1 Ceiling Treatment Options

| Method | Airborne STC | Impact IIC | Description | Height Loss |
| --- | --- | --- | --- | --- |
| Resilient channels + single drywall | STC +8-10 | IIC +3-5 | Metal channels between joists and drywall | 25 mm |
| Resilient channels + double drywall + Green Glue | STC +15-20 | IIC +5-8 | Maximum ceiling treatment from below | 50 mm |
| Independent ceiling (hat channel + isolation clips) | STC +15-18 | IIC +5-8 | Clips decouple entire ceiling assembly | 50-75 mm |
| Soffit ceiling (dropped ceiling with insulation) | STC +10-12 | IIC +3-5 | New ceiling hung below joists with insulation above | 100-200 mm |
| Direct-mount acoustic panels (from below) | STC +2-3 | IIC +0-2 | Only reduces echo; does not block transmission | 25-50 mm |

### 5.2 Ceiling Soundproofing — Step-by-Step

| Step | Action | Detail |
| --- | --- | --- |
| 1 | Install insulation above ceiling | Fill joist bays with rock wool (density ≥ 60 kg/m³) |
| 2 | Seal penetrations | Acoustic sealant around all ceiling penetrations (lights, vents, pipes) |
| 3 | Install resilient channels or isolation clips | Fasten perpendicular to joists; 400-600 mm spacing |
| 4 | First drywall layer | 12.5 mm drywall screwed to channels (not joists); use fine-thread drywall screws |
| 5 | Apply Green Glue | Two beads per sheet in serpentine pattern |
| 6 | Second drywall layer | 12.5 mm drywall; stagger seams relative to first layer |
| 7 | Seal perimeter | Acoustic sealant at all wall-ceiling joints |
| 8 | Finish | Tape, mud, paint |

### 5.3 Important: Resilient Channel Installation

| Do | Do Not |
| --- | --- |
| Install channels perpendicular to joists | Install channels parallel to joists |
| Use only one screw per channel per joist (allow flex) | Over-fasten channels (defeats decoupling) |
| Leave 12 mm gap from channel ends to walls | Let channels touch walls or other rigid surfaces |
| Use channels rated for ceiling load | Use damaged or bent channels |

## 6. Door and Window Sealing

### 6.1 Door Soundproofing

| Element | Solution | STC Improvement |
| --- | --- | --- |
| Hollow-core door | Replace with solid-core door (min 45 mm thick) | +10-15 STC |
| Door gaps (top and sides) | Install compression weatherstripping or magnetic seal | +3-5 STC |
| Door bottom gap | Install automatic door bottom seal (drops when door closes) | +5-8 STC |
| Door frame gaps | Seal frame-to-wall gap with acoustic sealant during installation | +2-3 STC |
| Door threshold | Install raised threshold with rubber seal | +3-5 STC |

### 6.2 Door Sound Ratings

| Door Type | Typical STC | Notes |
| --- | --- | --- |
| Hollow-core interior door | STC 20-25 | Poor sound blocking |
| Solid-core interior door (45 mm) | STC 30-35 | Significant improvement |
| Solid-core + weatherstripping + seal | STC 35-40 | Near-best for interior |
| Specialized acoustic door | STC 40-50 | Commercial-grade, expensive |

### 6.3 Window Soundproofing

| Solution | STC Improvement | Cost | Description |
| --- | --- | --- | --- |
| Window weatherstripping (seal gaps) | +2-5 STC | Low | Seal operable gaps with compression weatherstripping |
| Heavy curtains / acoustic drapes | +2-4 STC | Low-Medium | Floor-to-ceiling, overlapping wall |
| Window inserts (secondary glazing) | +5-10 STC | Medium | Acrylic or glass panel mounted inside existing frame |
| Replace with double-pane IGU | +5-8 STC | Medium-High | Insulated glass unit with different glass thicknesses |
| Replace with laminated glass | +8-12 STC | High | PVB interlayer between glass panes dampens sound |
| Storm window (exterior) | +5-8 STC | Medium | Additional window on exterior side |

### 6.4 Window Glass Comparison for Sound

| Glass Type | STC Rating | Description |
| --- | --- | --- |
| Single pane (3 mm) | STC 26-27 | Basic, poor sound blocking |
| Double pane IGU (3-12-3) | STC 32-35 | Standard insulated glass |
| Double pane IGU with laminated outer pane | STC 36-40 | Best balance of thermal and acoustic |
| Double pane IGU with different glass thicknesses (e.g., 4-12-6) | STC 35-38 | Different thicknesses reduce resonance |
| Triple pane IGU | STC 34-36 | Better thermal, but acoustic benefit limited vs. double |
| Laminated glass (6.4 mm) | STC 35-37 | PVB interlayer provides damping |

## 7. Materials Comparison

### 7.1 Insulation Materials (for cavities)

| Material | Density | STC Contribution | Fire Rating | Moisture Resistance | Cost (per m²) | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Rock wool (mineral wool) | 60-120 kg/m³ | High | Non-combustible (A1) | Water-resistant, does not rot | ¥30-80 | Recommended — best balance |
| Fiberglass | 10-30 kg/m³ | Moderate | Non-combustible | Absorbs water, loses effectiveness | ¥15-40 | Budget option; irritant during install |
| Polyester fiber | 30-50 kg/m³ | Moderate | Self-extinguishing | Moisture-resistant | ¥25-60 | Eco-friendly, safe to handle |
| Spray foam (closed-cell) | 35-50 kg/m³ | Low-Medium | Self-extinguishing | Excellent seal | ¥80-150 | Great air seal; poor acoustic absorber |
| Cellulose (blown) | 30-50 kg/m³ | Moderate | Treated fire-retardant | Absorbs moisture | ¥20-50 | Good for existing cavities |

### 7.2 Mass and Damping Materials

| Material | Weight (kg/m²) | Function | Cost (per m²) | Notes |
| --- | --- | --- | --- | --- |
| 12.5 mm drywall | 10-12 | Mass barrier | ¥15-30 | Standard; use for both layers |
| 15 mm drywall | 12-15 | Mass barrier | ¥20-40 | Heavier, better STC |
| Mass-loaded vinyl (MLV) | 3-6 kg/m² | Mass + some damping | ¥40-100 | Flexible, easy to install |
| Green Glue (damping compound) | N/A | Damping | ¥30-50 per tube | Between two rigid layers |
| Cement board | 15-20 kg/m² | Mass | ¥25-50 | Heavy; use in wet areas |
| Lead sheet | 10-20 kg/m² | Mass | ¥200-500 | Best mass/thickness ratio; toxic handling |

### 7.3 Decoupling Hardware

| Product | Function | Load Capacity | Cost | Notes |
| --- | --- | --- | --- | --- |
| Resilient channel (metal) | Decouple drywall from studs/joists | 15-20 kg/m | ¥15-30/m | Most common; easy install |
| Hat channel (Z-channel) | Decouple ceiling drywall | 15-20 kg/m | ¥20-40/m | Used with isolation clips |
| Isolation clips + hat channel | Complete decoupling system | 20-30 kg/m² | ¥40-80/m² | Best ceiling decoupling |
| Rubber isolation pads | Decouple floating floor | 50-100 kg/m² | ¥20-50/m² | For floating floor construction |
| Neoprene strip | Decouple framing members | Varies | ¥10-20/m | Under sole plates, between studs |

### 7.4 Surface Absorption Materials

| Material | NRC | Function | Aesthetic | Notes |
| --- | --- | --- | --- | --- |
| Acoustic foam panels | 0.60-0.85 | Absorb mid-high frequencies | Functional (studio look) | Easy to install; various shapes |
| Fabric-wrapped acoustic panels | 0.70-0.95 | Absorb broad frequency range | Decorative | Best for living spaces |
| Perforated wood panels (over absorber) | 0.50-0.80 | Absorb with wood aesthetic | Warm, natural | Requires cavity with absorber behind |
| Acoustic ceiling tiles | 0.50-0.75 | Absorb room echo | Standard commercial look | Drop ceiling compatible |
| Heavy curtains | 0.40-0.60 | Absorb reflections | Decorative | Also provide light blocking |

### 7.5 Material Selection Guide by Application

| Application | Recommended Combination | Estimated Cost (per m²) |
| --- | --- | --- |
| Bedroom wall | Staggered studs + rock wool + double drywall + Green Glue | ¥150-250 |
| Home theater wall | Double stud wall + rock wool + double drywall + MLV + Green Glue | ¥250-400 |
| Apartment floor (impact) | Rubber underlayment + floating floor + laminate | ¥100-200 |
| Apartment ceiling | Rock wool between joists + resilient channel + double drywall + Green Glue | ¥120-200 |
| Interior room dividers | Double drywall each side + rock wool + acoustic sealant | ¥80-150 |
| Budget retrofit wall | Add 1 layer drywall + Green Glue over existing wall | ¥50-100 |

## 8. Flanking Sound Prevention

### 8.1 Common Flanking Paths

| Path | Description | Prevention |
| --- | --- | --- |
| Electrical boxes back-to-back | Outlet boxes on opposite sides of wall share same cavity | Stagger outlets; use putty pads |
| HVAC ducts | Ducts carry sound between rooms | Use lined ducts; add sound traps |
| Plumbing chases | Pipes in shared chases transmit sound | Wrap pipes with insulation; seal penetrations |
| Floor joist continuity | Joists span both rooms, transmitting impact sound | Use floating floor or resilient ceiling |
| Window-to-window | Sound travels around wall through adjacent windows | Maintain wall mass between windows |
| Shared soffits | Bulkheads connecting rooms above ceiling | Build separate soffits; seal joints |

### 8.2 Sealing Checklist

- All penetrations sealed with acoustic sealant (not standard caulk)

- Electrical boxes: putty pads or isolation boxes installed

- Top and bottom plates sealed to floor/ceiling with acoustic sealant

- HVAC ducts: flexible connections, lined interior, sound traps

- Plumbing: rubber grommets at all pipe penetrations

- Door perimeters: compression weatherstripping on all three sides + automatic bottom seal

- Window perimeters: foam backer rod + acoustic sealant between frame and rough opening

## 9. Installation Quality Checklist

### 9.1 Wall Soundproofing

- Stud spacing correct (400 or 600 mm on center)

- Cavity insulation fits snugly; no gaps or compression

- All penetrations sealed with acoustic sealant

- Resilient channels installed perpendicular to framing

- Drywall seams staggered between layers

- Perimeter sealed with acoustic sealant (no standard caulk)

- No screw penetration through resilient channel into framing (except at ends)

### 9.2 Ceiling Soundproofing

- Rock wool fills joist bays completely

- Isolation clips or resilient channels securely fastened

- Drywall attached to channels only, not to joists

- Two layers of drywall with Green Glue between

- Perimeter acoustic sealant applied

- Light fixtures: use recessed cans with airtight seals or surface-mount

### 9.3 Floor Soundproofing

- Underlayment covers entire floor area; seams taped

- Perimeter isolation strip installed (floating floor)

- No rigid contact between floating floor and walls

- Floor height transition accommodated at doorways

- Underlayment rated for expected load

## 10. Quality Acceptance Criteria

| Item | Standard |
| --- | --- |
| Wall STC (bedroom partition) | ≥ 50 (tested or calculated) |
| Floor IIC (multi-story) | ≥ 50 (tested or calculated) |
| Perimeter sealing | No visible gaps; acoustic sealant at all joints |
| Cavity insulation | No voids or gaps visible during inspection |
| Drywall layers | Minimum two layers on sound-rated walls; seams staggered |
| Door seals | All three sides weatherstripped; automatic bottom seal functional |
| Decoupling | No rigid contact between decoupled layers and structure |
| Flanking paths | All identified paths addressed and sealed |

## 11. Common Mistakes to Avoid

| Mistake | Consequence | Prevention |
| --- | --- | --- |
| Using only acoustic foam on walls | Does not block sound transmission; only reduces echo | Use mass + decoupling + absorption together |
| Standard caulk instead of acoustic sealant | Standard caulk hardens and cracks, losing seal | Use acoustic sealant (remains flexible) |
| Compressing insulation in cavity | Reduces acoustic absorption; creates gaps | Cut to fit; do not compress |
| Over-fastening resilient channels | Channels become rigid, defeating decoupling | One screw per channel per joist |
| Back-to-back electrical boxes | Creates direct sound path through wall | Stagger or use isolation boxes |
| Skipping perimeter seal | Sound leaks around edges of soundproof assembly | Seal all perimeter joints with acoustic sealant |
| Single layer drywall on sound wall | Insufficient mass for good STC | Always use minimum two layers |
| Ignoring HVAC flanking | Sound travels through ducts, bypassing walls | Use lined ducts and sound traps |

# 楼梯装修指南

## 楼梯类型

| 类型 | 特点 | 适用场景 |
| --- | --- | --- |
| 直跑楼梯 | 结构简单 | 空间充足 |
| L型楼梯 | 节省空间 | 转角空间 |
| U型楼梯 | 美观 | 别墅 |
| 螺旋楼梯 | 最省空间 | 小空间 |

## 尺寸标准

| 项目 | 标准尺寸 |
| --- | --- |
| 踏步宽度 | ≥26cm |
| 踏步高度 | 15-18cm |
| 楼梯宽度 | ≥90cm |
| 扶手高度 | ≥90cm |

## 材料选择

| 材料 | 特点 | 价格 |
| --- | --- | --- |
| 实木 | 质感好 | 高 |
| 钢结构 | 现代感 | 中高 |
| 混凝土 | 结实 | 中 |
| 玻璃 | 通透 | 高 |

## 安全要求

- 踏步防滑

- 扶手牢固

- 照明充足

- 栏杆间距≤11cm

# 收纳设计指南

## 收纳规划原则

### 二八原则

- 80% 隐藏收纳（柜体内部）

- 20% 展示收纳（开放格）

### 动线原则

- 就近收纳：物品放在使用位置附近

- 按使用频率：常用物品放在易取位置

## 各空间收纳方案

### 玄关

- 鞋柜：底部留空放日常用鞋

- 挂衣区：外套、包包

- 换鞋凳

### 客厅

- 电视柜：隐藏电线和杂物

- 沙发边几：遥控器、杂志

- 角落储物柜

### 厨房

- 地柜：锅具、碗碟

- 吊柜：不常用物品

- 拉篮：调味用品

- 抽屉：餐具

### 卧室

- 衣柜：挂衣区+叠放区+抽屉

- 床头柜：日常用品

- 床下收纳：换季物品

### 卫生间

- 浴室柜：清洁用品

- 镜柜：洗漱用品

- 壁龛：洗浴用品

## 收纳柜尺寸参考

| 柜体 | 深度 | 高度 |
| --- | --- | --- |
| 鞋柜 | 35cm | 底部留空15cm |
| 衣柜 | 60cm | 到顶 |
| 橱柜 | 60cm | 地柜85cm |
| 电视柜 | 40cm | 40-50cm |
| 书柜 | 30cm | 到顶 |

## 收纳设计要点

- 柜体到顶：避免积灰

- 活动层板：灵活调整

- 抽屉比隔板好用

- 预留未来收纳空间

# 书房装修指南

## 布局设计

### 书桌摆放

- 靠窗采光好

- 桌面深度60cm

- 椅子后方留通道≥80cm

### 书柜设计

- 深度30cm

- 到顶设计

- 活动层板

## 灯光设计

- 主灯：吸顶灯，4000K

- 台灯：桌面照度300-500lux

- 书柜灯：LED灯带

## 色彩搭配

- 主色：白色或浅灰色

- 辅助色：原木色

- 点缀色：深蓝色或墨绿色

## 材料选择

- 地板：实木复合或木地板

- 墙面：乳胶漆

- 窗帘：遮光50-70%

## 隔音设计

- 门选择隔音门

- 窗户用双层中空玻璃

- 墙面可做隔音处理

# 别墅装修指南

## 别墅装修特点

- 面积大、空间多

- 功能需求复杂

- 预算较高

- 施工周期长

## 设计要点

### 功能规划

- 一层：客厅、餐厅、厨房、客房

- 二层：主卧套房、次卧

- 三层/阁楼：书房、影音室、健身室

- 地下室：储藏室、酒窖、保姆间

### 风格选择

- 新中式：传统与现代结合

- 欧式：奢华大气

- 现代：简洁时尚

## 系统配置

### 暖通系统

- 中央空调

- 地暖

- 新风系统

### 智能系统

- 全屋智能

- 安防系统

- 影音系统

### 水处理系统

- 全屋净水

- 软水系统

- 热水循环

## 预算参考

| 面积 | 品质型 | 豪华型 |
| --- | --- | --- |
| 200㎡ | 50-80万 | 80-150万 |
| 300㎡ | 80-120万 | 120-200万 |
| 500㎡ | 120-200万 | 200万+ |

## 注意事项

- 选择有别墅经验的装修公司

- 提前规划各系统

- 注意防水防潮

- 考虑后期维护

# 墙面瓷砖铺贴工艺 — Wall Tile Installation Craft

## 1. Overview

Wall tile installation requires careful surface preparation, precise layout planning, and proper adhesive and grout techniques. This guide covers the complete process from wall preparation through final grout application and expansion joint treatment for residential ceramic, porcelain, and natural stone wall tiles.

## 2. Wall Surface Preparation

### 2.1 Substrate Requirements

| Substrate Type | Preparation Required | Max Tolerance |
| --- | --- | --- |
| Concrete (cast-in-place) | Clean, remove form release agents; roughen smooth surfaces | 3 mm over 2 m straightedge |
| Cement block / brick | Plaster with cement mortar; cure 28 days minimum | 3 mm over 2 m |
| Cement backer board | Tape and mud joints; waterproof in wet areas | 2 mm over 2 m |
| Existing tile | Verify adhesion; score surface; apply bonding primer | Flatness of existing surface |
| Drywall (non-wet areas) | Verify water-resistant type in bathrooms | 2 mm over 2 m |
| Gypsum board (dry areas) | Standard drywall acceptable for light tile | 2 mm over 2 m |

### 2.2 Surface Preparation Checklist

- Remove all loose material, dust, oil, and debris

- Repair cracks and holes with cement-based patching compound

- Verify surface flatness: ≤ 3 mm deviation over 2 m

- Check plumb: ≤ 2 mm deviation over 3 m

- Apply waterproofing membrane in wet areas (shower walls, behind sinks)

- Waterproofing should extend: 1.8 m high in shower, 0.3 m above sink, 150 mm beyond tub edge

- Allow waterproofing to cure per manufacturer (typically 24-48 hours)

- Apply primer / bonding agent if required by substrate or adhesive type

### 2.3 Waterproofing (Wet Areas)

| Area | Minimum Coverage | Membrane Type |
| --- | --- | --- |
| Shower walls | 1.8 m minimum (to ceiling preferred) | Liquid-applied or sheet membrane |
| Bathtub surround | 150 mm above tub rim on all sides | Liquid-applied or sheet membrane |
| Behind bathroom sink | 0.3 m above counter | Liquid-applied membrane |
| Kitchen backsplash | Behind backsplash area | Not required (non-wet) |

Critical Detail: At inside corners (wall-to-wall, wall-to-floor), install pre-formed waterproofing corners or embed fabric into liquid membrane at corners.

## 3. Tile Material Selection

### 3.1 Tile Types for Walls

| Type | Water Absorption | Weight | Best For | Notes |
| --- | --- | --- | --- | --- |
| Ceramic (ceramic tile) | 3-10% | Light | Dry areas, low-cost | Easy to cut, wide variety |
| Porcelain (porcelain tile) | < 0.5% | Medium-Heavy | Wet areas, durable | Harder to cut, requires diamond blade |
| Natural stone (marble, travertine) | Varies | Heavy | Luxury aesthetics | Requires sealing, natural variation |
| Glass tile | 0% | Light | Accent walls, backsplash | Requires special adhesive, transparent adhesive |
| Mosaic (sheet-mounted) | Varies | Light-Medium | Accent areas, curves | Pre-mounted on mesh sheets |
| Large-format (≥ 600 mm) | < 0.5% | Heavy | Modern, seamless look | Requires premium adhesive, leveling system |

### 3.2 Adhesive Selection

| Tile Type | Tile Size | Adhesive Type | Coverage |
| --- | --- | --- | --- |
| Ceramic (small) | ≤ 300 mm | Cement-based thin-set (C1) | 3-4 mm bed |
| Porcelain | Any | Polymer-modified thin-set (C2) | 4-6 mm bed |
| Large-format (≥ 600 mm) | ≥ 600 mm | Premium polymer-modified (C2TE S1) | 6-10 mm bed, back-butter required |
| Natural stone | Any | White polymer-modified (prevents staining) | 4-8 mm bed |
| Glass tile | Any | Glass tile adhesive (white, non-sag) | Per manufacturer |
| Mosaic | Any | Polymer-modified thin-set (C2) | 3-4 mm bed |

Adhesive Classification (EN 12004): | Code | Meaning | |——|———| | C1 | Normal cementitious adhesive | | C2 | Improved cementitious adhesive (higher bond strength) | | T | Thixotropic / non-slip (for walls) | | E | Extended open time | | S1 | Deformable (flexible) | | S2 | Highly deformable (very flexible) |

Minimum Recommendation for Wall Tiles: C2T (improved, non-slip cementitious adhesive).

## 4. Layout Planning

### 4.1 Layout Principles

- Avoid narrow slivers — no cut tile narrower than half a full tile at edges.

- Center on focal points — align layout around the most visible area (e.g., shower niche, behind faucet).

- Horizontal lines should be level; vertical lines should be plumb.

- Pattern alignment — ensure patterns (if any) flow correctly across walls and corners.

- Account for accessories — plan cuts around outlets, switches, shower valves, and towel bars.

### 4.2 Layout Calculation

Number of full tiles = Wall width ÷ (Tile width + Grout width)
Edge cut = Remaining width ÷ 2 (for centered layout)

Example: 2400 mm wall, 300 mm tile, 2 mm grout - Full tiles per row: 2400 ÷ 302 = 7.94 → 7 full tiles - Remaining: 2400 - (7 × 302) = 186 mm - Edge cuts: 186 ÷ 2 = 93 mm (acceptable — more than half tile)

### 4.3 Dry Layout Procedure

| Step | Action |
| --- | --- |
| 1 | Snap a level reference line at the height of the second row (first row = full tiles above floor) |
| 2 | Lay a temporary ledger board (straight batten) along this line |
| 3 | Dry-lay tiles from center outward on both sides |
| 4 | Adjust starting point if edge cuts are too narrow |
| 5 | Mark tile positions on wall with pencil |
| 6 | Plan cuts for corners, outlets, and fixtures |

### 4.4 Common Layout Patterns

| Pattern | Description | Waste Factor | Best For |
| --- | --- | --- | --- |
| Straight / Stack bond | Grid alignment, simplest | 5-10% | Most walls |
| Running bond / Brick | Offset 50% each row | 10-15% | Subway tile, traditional |
| Diagonal (45 degrees) | Tiles rotated 45 degrees | 15-20% | Visual interest |
| Herringbone | V-shaped interlocking pattern | 20-25% | Accent walls, luxury |
| Staggered (1/3 offset) | Offset by one-third | 10% | Rectangular tiles |

## 5. Tile Cutting

### 5.1 Cutting Methods

| Method | Tile Types | Cut Types | Accuracy |
| --- | --- | --- | --- |
| Manual snap cutter | Ceramic, small porcelain | Straight cuts | High |
| Wet saw (diamond blade) | All types | Straight, diagonal, L-cuts | Very High |
| Angle grinder + diamond blade | All types | Curves, notches, holes | Moderate (skill-dependent) |
| Hole saw / core bit | All types | Round holes (for pipes, valves) | High |
| Tile nippers | Ceramic, mosaic | Small curves, nibbles | Moderate |
| Oscillating tool | All types | Outlet boxes, precise cutouts | High |

### 5.2 Cutting Around Outlets and Fixtures

| Fixture | Cut Method | Tolerance |
| --- | --- | --- |
| Electrical outlet | Measure, mark, cut with hole saw or oscillating tool | ±2 mm |
| Shower valve | Use manufacturer template; cut with hole saw | Per valve escutcheon size |
| Pipe penetrations | Core drill or angle grinder notch | 3-5 mm clearance |
| Towel bar anchor | Drill through installed tile with carbide/glass bit | Exact anchor size |
| Niche recess | Plan niche location in layout; frame before tiling | Per niche dimensions |

### 5.3 Cutting Checklist

- Always measure twice, cut once

- Account for grout width in measurements

- Use water lubrication on wet saw to reduce dust and heat

- Support tile fully during cutting to prevent breakage

- Smooth cut edges with a rubbing stone

- For inside corners: cut one tile to butt into the corner, overlapping tile on the visible side

- For outside corners: use tile trim, miter cuts (45 degrees), or bullnose edge tiles

## 6. Tile Installation Process

### 6.1 Thin-Set / Adhesive Application

| Step | Action | Detail |
| --- | --- | --- |
| 1 | Mix adhesive | Per manufacturer ratios; use paddle mixer on drill; rest (slake) 5-10 minutes |
| 2 | Check consistency | Should hold a notch when troweled; not too runny or stiff |
| 3 | Apply to wall | Spread with flat side of trowel first to create a scratch coat |
| 4 | Comb adhesive | Use notched trowel at 45-60 degree angle; create uniform ridges |
| 5 | Back-butter large tiles | Apply thin skim coat to tile back with flat side of trowel (tiles ≥ 300 mm) |
| 6 | Set tile | Press firmly with slight twisting motion; seat into ridges |
| 7 | Check alignment | Use tile leveling system for large-format tiles |
| 8 | Adjust within open time | Tiles can be repositioned within 10-20 minutes (varies by adhesive) |

### 6.2 Notch Trowel Selection

| Tile Size | Recommended Notch Size | Bed Thickness |
| --- | --- | --- |
| ≤ 150 mm | 6 mm x 6 mm square notch | 3 mm |
| 150-300 mm | 8 mm x 8 mm square notch | 4 mm |
| 300-600 mm | 10 mm x 10 mm square notch | 5 mm |
| ≥ 600 mm (large-format) | 12 mm x 12 mm square notch | 6 mm+ (back-butter required) |
| Mosaic (sheet) | 4 mm x 4 mm square notch | 2 mm |

### 6.3 Adhesive Coverage Requirement

After setting a tile, pull one tile back to verify coverage:

| Tile Type | Minimum Coverage |
| --- | --- |
| Dry area (wall) | 80% |
| Wet area (wall) | 95% |
| Floor | 95% |
| Exterior | 100% |

If coverage is insufficient: increase trowel notch size, adjust angle, or back-butter the tile.

### 6.4 Tile Leveling System

For tiles ≥ 300 mm or large-format tiles, use a clip-and-wedge leveling system:

| Component | Function |
| --- | --- |
| Base clip | Slides under tile edges, straddles grout joint |
| Wedge/cap | Locks into clip, pulls tiles to same plane |
| Removal | Break clips off after adhesive cures (24 hours) |

Benefits: Eliminates lippage (height difference between adjacent tiles), ensures flat surface.

## 7. Grout Application

### 7.1 Grout Types

| Type | Composition | Joint Width | Pros | Cons | Best For |
| --- | --- | --- | --- | --- | --- |
| Cement-based (sanded) | Portland cement + sand | 3-12 mm | Strong, inexpensive, wide color range | Requires sealing, can stain | Floor, wide joints |
| Cement-based (unsanded) | Portland cement, no sand | 1-3 mm | Smooth finish, won’t scratch tile | Less durable | Wall, narrow joints, glass |
| Epoxy | Epoxy resin + filler | 1.5-12 mm | Waterproof, stain-proof, no sealing needed | Expensive, difficult to apply, fast set | Wet areas, kitchens, commercial |
| Urethane | Polyurethane-based | 1.5-12 mm | Flexible, stain-resistant, easy application | More expensive than cement | Residential, high-use areas |

### 7.2 Grout Joint Width

| Tile Type | Recommended Joint Width |
| --- | --- |
| Rectified tile (precise edges) | 1.5-2 mm |
| Standard pressed tile | 2-3 mm |
| Natural stone | 3-5 mm (accommodates variation) |
| Mosaic | Per mesh spacing (usually 1.5-2 mm) |
| Large-format (≥ 600 mm) | 2-3 mm |

### 7.3 Grout Application Steps

| Step | Action | Detail |
| --- | --- | --- |
| 1 | Wait for adhesive cure | Minimum 24 hours after tile setting |
| 2 | Remove spacers/leveling clips | Clean all debris from joints |
| 3 | Mix grout | Per manufacturer ratios; target peanut-butter consistency |
| 4 | Apply grout | Use rubber float at 45-degree angle; force grout into joints |
| 5 | Remove excess | Hold float at 90 degrees; scrape diagonally across tiles |
| 6 | Initial clean | Wait 15-20 minutes; wipe with damp (not wet) sponge in circular motion |
| 7 | Final clean | After grout hazes (1-2 hours), buff with dry microfiber cloth |
| 8 | Cure | Keep area dry for 48-72 hours; avoid showering |
| 9 | Seal (cement grout only) | Apply penetrating sealer after 7 days full cure |

### 7.4 Grout Color Selection

| Strategy | Effect |
| --- | --- |
| Match grout to tile | Seamless, unified look; hides grout lines |
| Contrast grout to tile | Emphasizes tile pattern; highlights layout |
| Mid-tone gray | Forgiving; hides dirt; works with most tiles |
| White | Clean look; shows dirt and stains easily |
| Dark (charcoal, black) | Dramatic; hides stains well |

## 8. Expansion Joints (伸缩缝)

### 8.1 Where Expansion Joints Are Required

| Location | Joint Width | Material |
| --- | --- | --- |
| Wall-to-floor intersection | 3-5 mm | Flexible silicone sealant (color-matched) |
| Wall-to-ceiling intersection | 3-5 mm | Flexible silicone sealant |
| Inside corners (wall-to-wall) | 2-3 mm | Silicone sealant (do NOT grout corners) |
| Every 3-5 m of continuous tile field | 3-5 mm | Flexible sealant or pre-formed expansion strip |
| Around penetrating pipes/fixtures | 3 mm | Silicone sealant |
| Change of substrate (e.g., drywall to cement board) | 3 mm | Flexible sealant |
| Threshold transitions | Per transition profile | Transition strip or sealant |

### 8.2 Why Not to Grout Corners

Grout is rigid and will crack at inside and outside corners due to: - Building settlement and movement - Thermal expansion and contraction - Substrate deflection

Always use flexible silicone sealant at corners, color-matched to grout for a seamless appearance.

### 8.3 Sealant Application at Joints

| Step | Action |
| --- | --- |
| 1 | Clean joint: remove dust, debris, and moisture |
| 2 | Insert foam backer rod for joints > 5 mm deep |
| 3 | Apply masking tape along both sides of joint (for clean edges) |
| 4 | Apply silicone sealant with caulk gun; fill joint slightly proud |
| 5 | Tool the joint with a wet finger or tooling tool for concave profile |
| 6 | Remove masking tape immediately |
| 7 | Allow to cure per manufacturer (typically 24 hours) |

## 9. Quality Acceptance Criteria

| Item | Standard |
| --- | --- |
| Flatness | ≤ 2 mm deviation over 2 m straightedge |
| Lippage (height difference) | ≤ 0.5 mm for joints ≤ 3 mm; ≤ 1 mm for joints > 3 mm |
| Plumb (vertical alignment) | ≤ 2 mm deviation over 3 m |
| Grout joint consistency | ±0.5 mm of specified width throughout |
| Hollow sound (tap test) | No hollow spots > 5% of tile area; no hollow spots at corners/edges |
| Surface defects | No visible cracks, chips, or stains under normal lighting |
| Corner joints | Sealed with flexible sealant, not grout |
| Waterproofing (wet areas) | Pass flood test or moisture meter verification |

## 10. Common Mistakes to Avoid

| Mistake | Consequence | Prevention |
| --- | --- | --- |
| Tiling on unlevel walls | Visible lippage, uneven grout lines | Level substrate before starting |
| Skipping waterproofing in wet areas | Water damage behind tiles | Apply membrane before tiling |
| Using wrong adhesive for tile type | Tile delamination, failure | Match adhesive to tile and substrate |
| Not back-buttering large tiles | Inadequate bond, hollow spots | Always back-butter tiles ≥ 300 mm |
| Grouting before adhesive cures | Tiles shift, grout cracks | Wait 24 hours minimum |
| Using sanded grout on narrow joints | Incomplete joint fill, rough surface | Use unsanded for joints < 3 mm |
| No expansion joints | Cracked tiles and grout from movement | Plan and install all expansion joints |
| Starting with partial tile at bottom | Visible narrow row at floor | Use ledger board; cut bottom row to fit |

# 全屋净水系统

## 系统组成

| 设备 | 位置 | 作用 | 价格 |
| --- | --- | --- | --- |
| 前置过滤器 | 入户总管 | 过滤大颗粒杂质 | 200-800 |
| 中央净水机 | 总管 | 去除余氯、有机物 | 2000-6000 |
| 中央软水机 | 总管 | 软化水质 | 3000-8000 |
| 末端净水机 | 厨房 | 直饮水 | 1000-5000 |
| 管线机 | 厨房 | 即热饮水 | 1000-3000 |

## 选择建议

### 基本配置

- 前置过滤器 + 末端净水机

### 完整配置

- 前置 + 中央净水 + 中央软水 + 末端净水

## 安装要点

- 预留安装空间

- 预留电源

- 预留排水口

- 在水电阶段确定位置

## 维护

| 设备 | 维护周期 | 维护内容 |
| --- | --- | --- |
| 前置过滤器 | 每月 | 冲洗 |
| 中央净水 | 每年 | 更换滤芯 |
| 中央软水 | 每月 | 加盐 |
| 末端净水 | 每半年 | 更换滤芯 |

# 卫生间通风排气工艺 — Bathroom Ventilation Craft

## 1. Overview

Proper bathroom ventilation is critical for moisture control, odor elimination, mold prevention, and indoor air quality. This guide covers exhaust fan sizing, duct routing, ventilation strategies, humidity management, and odor prevention for residential bathrooms.

## 2. Why Ventilation Matters

| Risk Without Ventilation | Consequence |
| --- | --- |
| Humidity accumulation | Mold growth on walls, ceiling, grout, and silicone |
| Odor buildup | Unpleasant living environment, lingering smells |
| Stagnant air | Elevated CO₂, VOCs from cleaning products |
| Condensation | Damage to paint, wallpaper, cabinetry, and fixtures |
| Structural damage | Rot in wall framing, ceiling joists over time |

Building Code Reference: Most residential building codes require a minimum of 50 CFM (85 m³/h) continuous ventilation for bathrooms, or 200 CFM (340 m³/h) intermittent ventilation during use.

## 3. Ventilation Methods

### 3.1 Natural Ventilation (Window)

| Factor | Detail |
| --- | --- |
| Applicability | Bathrooms with operable windows |
| Minimum window area | 5% of floor area (for ventilation-only) |
| Effectiveness | Good for light moisture; insufficient for heavy showers |
| Limitations | Weather-dependent, privacy concerns, energy loss in winter/summer |

Checklist — When a Window Is Sufficient: - [ ] Bathroom has an operable window - [ ] Window area ≥ 0.5 m² - [ ] Climate is dry (relative humidity < 60% average) - [ ] Shower frequency is light (1-2 per day) - [ ] No adjacent rooms at risk from moisture migration

### 3.2 Mechanical Ventilation (Exhaust Fan)

| Factor | Detail |
| --- | --- |
| Applicability | All bathrooms, mandatory for windowless bathrooms |
| Effectiveness | Excellent — controlled airflow regardless of weather |
| Types | Ceiling-mounted, wall-mounted, inline (remote), combination fan/light/heater |
| Noise | 0.5-6.0 sones (lower is quieter) |

Recommendation: Even bathrooms with windows should have mechanical exhaust for reliable moisture control, especially in humid climates or during winter when windows stay closed.

### 3.3 Comparison

| Criterion | Natural | Mechanical | Combined (Best) |
| --- | --- | --- | --- |
| Moisture removal | Moderate | Excellent | Excellent |
| Odor removal | Good | Excellent | Excellent |
| Energy cost | Free | ¥30-80/month | ¥30-80/month |
| Noise | None | 1.0-4.0 sones (quality units) | 1.0-4.0 sones |
| Year-round use | Limited | Yes | Yes |
| Installation cost | N/A (existing window) | ¥300-1,500 | ¥300-1,500 |

## 4. Exhaust Fan Sizing

### 4.1 Sizing by Air Changes Per Hour (ACH)

The standard for residential bathrooms is 8 air changes per hour (ACH).

Formula:

Required CFM = (Bathroom Volume in ft³ × 8 ACH) ÷ 60 minutes
Required m³/h = Bathroom Volume (m³) × 8 ACH

### 4.2 Quick Sizing Table

| Bathroom Size | Floor Area (m²) | Ceiling Height | Volume (m³) | Min Exhaust (m³/h) | Recommended Fan (m³/h) |
| --- | --- | --- | --- | --- | --- |
| Small | 3-4 | 2.7 m | 8-11 | 64-88 | 100 |
| Medium | 5-7 | 2.7 m | 14-19 | 108-148 | 150 |
| Large | 8-12 | 2.7 m | 22-32 | 172-256 | 200-250 |
| Luxury (with tub) | 12-20 | 2.7 m | 32-54 | 256-432 | 300-400 |

### 4.3 Sizing by Fixture Count

| Fixture | Additional CFM Required |
| --- | --- |
| Toilet | 50 CFM (85 m³/h) |
| Shower | 50 CFM (85 m³/h) |
| Bathtub | 50 CFM (85 m³/h) |
| Jetted tub | 100 CFM (170 m³/h) |
| Each additional fixture | +50 CFM (+85 m³/h) |

Rule: Size the fan for the greater of the ACH calculation or the fixture-based calculation.

### 4.4 Fan Type Selection

| Type | Installation | Noise | Best For |
| --- | --- | --- | --- |
| Ceiling-mounted | Drop ceiling / above ceiling space | Low-Medium | Standard bathrooms |
| Wall-mounted | Direct exterior wall | Medium | Retrofits, no ceiling access |
| Inline (remote) | In duct run, outside bathroom | Very low (fan is remote) | Luxury bathrooms, noise-sensitive |
| Fan/light/heater combo | Ceiling-mounted | Varies | Small bathrooms, multi-function |
| Heat recovery ventilator (HRV) | Dedicated unit | Low | Whole-house ventilation |

## 5. Duct Routing

### 5.1 Routing Principles

- Shortest path to exterior — minimize duct length to reduce static pressure loss.

- Fewest bends — each 90-degree elbow adds equivalent resistance of ~3 m of straight duct.

- Never terminate into an attic, soffit, or crawl space — must exhaust directly outdoors.

- Rigid duct preferred over flexible duct — rigid smooth-wall duct has ~50% less airflow resistance.

- Slope horizontal runs slightly downward toward the exterior to prevent condensate drip-back.

### 5.2 Duct Material Comparison

| Material | Airflow Efficiency | Durability | Cost | Recommended |
| --- | --- | --- | --- | --- |
| Rigid PVC (110 mm) | Excellent | High | Moderate | Yes — preferred |
| Rigid metal (galvanized) | Excellent | High | Moderate-High | Yes |
| Semi-rigid aluminum | Good | Medium | Low | Acceptable for short runs |
| Flexible plastic hose | Poor (corrugated) | Low | Very low | Avoid if possible |
| Flexible insulated | Moderate | Medium | High | Only when insulation needed |

### 5.3 Maximum Duct Length by Fan Power

| Fan Rating (m³/h) | Rigid Duct Max Length | With 1 Elbow | With 2 Elbows |
| --- | --- | --- | --- |
| 100 | 6 m | 4.5 m | 3 m |
| 150 | 10 m | 8 m | 6 m |
| 200 | 14 m | 12 m | 9 m |
| 250 | 18 m | 15 m | 12 m |

### 5.4 Duct Installation Checklist

- Use rigid PVC or metal ducting (minimum 100 mm / 4” diameter)

- Seal all joints with aluminum foil tape (not cloth duct tape)

- Install a backdraft damper at the exterior termination

- Use exterior wall cap with bird screen

- Insulate duct if running through unconditioned space (cold climates)

- Slope horizontal runs 1/8” per foot (10 mm per meter) toward exterior

- Support duct every 1.2 m with hanger straps

- Verify no kinks or crushing during installation

## 6. Humidity Control

### 6.1 Target Humidity Levels

| Condition | Target RH |
| --- | --- |
| Normal bathroom use | 40-60% |
| During/after shower | Up to 80% (temporary, should drop within 20 minutes) |
| Long-term storage areas | Below 50% |
| Winter (cold climate) | 30-40% (to prevent window condensation) |

### 6.2 Humidity Control Strategies

| Strategy | Implementation | Effectiveness |
| --- | --- | --- |
| Timed exhaust fan | Run fan 20-30 minutes after shower ends | High |
| Humidity-sensing fan | Fan auto-activates when RH exceeds threshold | Very High |
| Continuous low-speed fan | Fan runs at 20-30 CFM continuously | High |
| Dehumidifier | Standalone unit for chronic humidity issues | High |
| Wipe-down routine | Squeegee glass, wipe surfaces after shower | Moderate |
| Ventilated mirror | Mirror with built-in fan or heating | Supplemental |

### 6.3 Recommended Fan Controls

| Control Type | Description | Cost | Best For |
| --- | --- | --- | --- |
| Manual switch | Standard wall switch | ¥20-50 | Basic installations |
| Timer switch | Fan runs for preset duration (10-60 min) | ¥80-200 | Most bathrooms |
| Humidity sensor | Auto-activates at set RH threshold (e.g., 60%) | ¥150-400 | High-use bathrooms |
| Motion sensor | Activates on entry; delays shutoff | ¥150-350 | Guest bathrooms |
| Smart controller | App/WiFi control, scheduling, humidity monitoring | ¥200-600 | Smart home integration |

Best Practice: Install a timer or humidity-sensing switch to ensure the fan runs long enough after bathing — this is the most common cause of mold in bathrooms.

## 7. Odor Prevention

### 7.1 Sources and Solutions

| Odor Source | Root Cause | Solution |
| --- | --- | --- |
| Toilet | Dry P-trap, poor seal | Ensure P-trap always has water; replace wax ring if needed |
| Floor drain | Dry water seal | Pour water into unused floor drains weekly; install trap primer |
| Shower drain | Biofilm, hair buildup | Clean drain monthly; use enzyme cleaner |
| Exhaust fan duct | Mold growth in duct | Clean duct annually; ensure proper slope to exterior |
| Ventilation backflow | No backdraft damper | Install backdraft damper at exterior termination |
| Wall/floor grout | Mold and mildew | Improve ventilation; deep clean and reseal grout |

### 7.2 Backflow Prevention

A backdraft damper is essential to prevent: - Odors from neighboring units (in multi-story buildings) - Outdoor pollutants entering through the duct - Cold/hot outdoor air infiltrating when fan is off

Types: | Type | Mechanism | Pros | Cons | |——|———–|——|——| | Gravity flap | Weighted flap closes by gravity | Simple, reliable, cheap | May not seal perfectly | | Spring-loaded | Spring closes flap tightly | Better seal | Spring can wear out | | Motorized | Powered damper syncs with fan | Best seal, quiet | Higher cost, electrical connection |

### 7.3 Floor Drain Maintenance

| Frequency | Action |
| --- | --- |
| Weekly | Pour 200 ml water into rarely used floor drains |
| Monthly | Clean drain grate, remove hair/debris |
| Quarterly | Pour enzyme-based drain cleaner |
| Annually | Inspect P-trap and replace damaged components |

## 8. Special Considerations

### 8.1 Windowless Bathrooms (暗卫)

| Requirement | Solution |
| --- | --- |
| Minimum exhaust | 150 m³/h fan, continuous or humidity-controlled |
| Supplemental lighting | Bright LED (≥500 lux) to compensate for no natural light |
| Moisture monitoring | Digital hygrometer to track RH levels |
| Door undercut | 12-15 mm gap at bottom for makeup air |
| Material selection | Use moisture-resistant paint, cement board, porcelain tile |

### 8.2 Multi-Story Building Ventilation

| Issue | Prevention |
| --- | --- |
| Odor transfer between floors | Individual exhaust to exterior (not shared shaft without backflow prevention) |
| Shared ventilation shaft | Install check valve on each unit’s connection |
| Negative pressure | Ensure makeup air path exists (door undercut, transfer grille) |

### 8.3 Winter Ventilation

| Challenge | Mitigation |
| --- | --- |
| Cold air infiltration | Install insulated duct + exterior wall cap with thermal break |
| Condensation in duct | Insulate duct runs through unheated spaces |
| Energy loss | Use heat recovery ventilator (HRV) or run fan on timer only |
| Frozen exterior cap | Use freeze-resistant wall cap in cold climates |

## 9. Installation Checklist

### 9.1 Pre-Installation

- Calculate required airflow (ACH method and fixture method)

- Select fan type and model

- Plan duct route (measure distance, count bends)

- Verify electrical circuit capacity (typically 15A circuit)

- Check local building code requirements

### 9.2 Installation

- Cut ceiling/wall opening per fan template

- Mount fan housing to joist or support bracket

- Connect duct to fan outlet with foil tape

- Route duct to exterior termination point

- Install exterior wall cap with backdraft damper

- Wire fan to switch or controller

- Install fan grille

- Test operation: verify airflow, check for noise/vibration

### 9.3 Post-Installation Verification

- Measure airflow with anemometer or flow hood

- Verify noise level is within acceptable range (< 3.0 sones recommended)

- Check for air leaks at all duct joints

- Confirm exterior damper opens when fan runs and closes when off

- Run moisture test: shower 10 minutes, verify mirror clears within 20 minutes after

## 10. Maintenance Schedule

| Frequency | Task |
| --- | --- |
| Monthly | Clean fan grille (vacuum or wash) |
| Every 6 months | Remove grille, clean fan blades and housing |
| Annually | Inspect duct for obstructions; clean if needed |
| Annually | Check exterior wall cap for debris, bird nests |
| Every 2-3 years | Replace fan motor if showing reduced airflow |
| As needed | Replace filters on combo units |

# 吊顶工艺详解

## 吊顶类型

| 类型 | 适用场景 | 价格（元/㎡） |
| --- | --- | --- |
| 轻钢龙骨石膏板 | 客厅、卧室 | 100-200 |
| 铝扣板 | 厨房、卫生间 | 80-150 |
| PVC扣板 | 厨卫 | 40-80 |
| 集成吊顶 | 厨卫 | 100-200 |
| 矿棉板 | 办公室 | 50-100 |

## 轻钢龙骨石膏板

### 材料

- 轻钢龙骨：主龙骨、副龙骨、吊杆

- 石膏板：9.5mm或12mm

- 配件：吊杆、膨胀螺栓

### 施工工艺

- 弹线确定高度

- 安装吊杆（间距≤100cm）

- 安装主龙骨（间距≤100cm）

- 安装副龙骨（间距≤40cm）

- 安装石膏板

- 接缝处理

- 防锈处理

### 关键指标

- 平整度≤3mm/2m

- 转角整板切割

- 重型灯具单独吊挂

## 铝扣板

### 规格

- 300×300mm

- 300×600mm

- 厚度0.6-0.8mm

### 安装要点

- 龙骨水平

- 扣板平整

- 电器模块匹配

## 验收要点

- 平整度≤3mm/2m

- 接缝均匀

- 电器安装牢固

- 边角收口美观

# 木工工艺详解

## 吊顶施工

### 轻钢龙骨石膏板

- 弹线确定高度

- 安装吊杆（间距≤100cm）

- 安装主龙骨（间距≤100cm）

- 安装副龙骨（间距≤40cm）

- 安装石膏板

- 接缝处理

- 防锈处理

### 铝扣板

- 安装边角

- 安装龙骨

- 安装扣板

## 柜体制作

### 现场制作流程

- 放线定位

- 制作框架

- 安装背板

- 安装门板

- 安装五金

### 工艺要点

- 板材切割精确

- 封边严密

- 铰链间距≤60cm

## 验收标准

| 项目 | 标准 |
| --- | --- |
| 吊顶平整度 | ≤3mm/2m |
| 石膏板接缝 | 错开 |
| 柜体牢固度 | 稳固不晃动 |

# 木工施工工艺流程

## 吊顶施工

### 施工流程

- 弹线 → 确定吊顶高度和造型

- 安装吊杆 → 间距≤100cm

- 安装龙骨 → 主龙骨间距≤100cm，副龙骨间距≤40cm

- 安装石膏板 → 错缝安装

- 接缝处理 → 嵌缝石膏+绷带

- 防锈处理 → 钉眼点防锈漆

### 工艺要点

- 转角处用整板切割

- 石膏板与墙面留缝 3-5mm

- 重型灯具单独吊挂

- 厨卫用铝扣板吊顶

## 柜体制作

### 施工流程

- 放线 → 确定柜体位置

- 制作框架 → 板材切割组装

- 安装背板 → 固定在墙面上

- 安装门板 → 调整缝隙

- 安装五金 → 铰链、滑轨

### 工艺要点

- 板材切割精确

- 封边严密

- 铰链间距≤60cm

- 门板缝隙均匀

## 验收要点

- 吊顶平整度≤3mm/2m

- 石膏板接缝错开

- 转角整板切割

- 柜体牢固

- 门板开合顺畅

- 五金安装齐全

# 水电工施工工艺流程

## 水路施工

### 施工流程

- 定位放线 → 根据设计图标注用水点位

- 开槽 → 墙面开槽，深度管径+10mm

- 布管 → PPR管热熔连接

- 固定 → 管卡固定，间距≤60cm

- 打压 → 0.8MPa 保压30分钟

- 封槽 → 水泥砂浆封堵

### 工艺要点

- 水管走顶不走地

- 冷热水管间距≥15cm

- 热水管包覆保温棉

- 管根做加强防水

## 电路施工

### 施工流程

- 定位放线 → 标注开关插座位置

- 开槽 → 墙面开槽，禁止横槽超过30cm

- 布管 → PVC线管敷设

- 穿线 → 电线穿管

- 安装底盒 → 固定底盒

- 测试 → 通路和绝缘测试

### 工艺要点

- 强弱电间距≥30cm

- 线管弯曲半径≥6倍管径

- 电线截面符合负荷要求

- 接头必须在底盒内

## 常见问题

| 问题 | 原因 | 后果 |
| --- | --- | --- |
| 水管渗漏 | 热熔连接不合格 | 墙面发霉 |
| 电线短路 | 电线受损 | 安全隐患 |
| 插座没电 | 接线错误 | 无法使用 |
| 水压不足 | 管径过小 | 使用不便 |

## 验收清单

- 打压测试合格

- 电路通路正常

- 绝缘电阻≥0.5MΩ

- 管线走向拍照记录

- 管卡间距符合要求

- 强弱电间距符合要求

# 电路布线详解

## 回路设计

### 标准回路

| 回路 | 电线 | 空开 | 用途 |
| --- | --- | --- | --- |
| 照明 | 1.5mm² | 16A | 全屋照明 |
| 插座 | 2.5mm² | 20A | 普通插座 |
| 空调 | 4mm² | 25A | 每台空调 |
| 厨房 | 4mm² | 25A | 厨房电器 |
| 卫生间 | 4mm² | 25A | 卫生间电器 |
| 冰箱 | 2.5mm² | 16A | 冰箱单独回路 |

## 布线方式

### 点对点

- 最短距离

- 省材料

- 检修困难

### 横平竖直

- 规范美观

- 检修方便

- 费材料

## 开关插座位置

### 开关高度

- 开关：130cm

- 插座：30cm

- 空调插座：180cm

- 厨房插座：120cm

### 插座数量建议

| 空间 | 插座数量 |
| --- | --- |
| 客厅 | 6-8个 |
| 卧室 | 4-6个 |
| 厨房 | 6-8个 |
| 卫生间 | 2-3个 |
| 书房 | 4-6个 |

## 注意事项

- 强弱电分离

- 电线穿管

- 接头在底盒内

- 拍照记录

# 安装工施工工艺流程

## 地板安装

### 实木地板

- 地面找平 → 2m靠尺检查≤3mm

- 打龙骨 → 间距≤30cm

- 铺防潮垫 → 重叠20cm

- 铺地板 → 留缝8-12mm

- 安装踢脚线 → 紧贴墙面

### 复合地板

- 地面找平 → 2m靠尺检查≤3mm

- 铺防潮垫 → 满铺

- 铺地板 → 锁扣连接

- 安装踢脚线

## 门安装

### 施工流程

- 安装门框 → 校正垂直度

- 固定 → 发泡胶填充

- 安装门扇 → 调整缝隙

- 安装五金 → 合页、门锁、门吸

### 工艺要点

- 门框垂直度≤2mm/2m

- 门扇与地面间隙5-8mm

- 合页数量≥3个

- 发泡胶固化后修边

## 洁具安装

### 马桶安装

- 检查排污管

- 安装法兰圈

- 放置马桶

- 打胶密封

- 连接水箱

### 花洒安装

- 确定安装高度（混水阀距地90-100cm）

- 安装混水阀

- 安装花洒杆

- 连接软管

- 测试

## 灯具安装

### 施工流程

- 断电

- 安装底座

- 接线（火线、零线、地线）

- 固定灯体

- 通电测试

### 注意事项

- 重型灯具必须单独吊挂

- 接线正确

- 防水区域用防水灯具

## 验收要点

- 地板平整无异响

- 门开合顺畅

- 洁具不渗漏

- 灯具正常

- 开关插座通电

- 五金安装牢固

# 厨房台面工艺 — Kitchen Countertop Fabrication & Installation

## 1. Overview

Kitchen countertops are one of the most heavily used surfaces in a home. This guide covers material selection, edge profiling, seam treatment, sink cutouts, installation procedures, and long-term maintenance for residential kitchen countertops.

## 2. Material Comparison

### 2.1 Quartz Stone (石英石)

| Attribute | Detail |
| --- | --- |
| Composition | ~90-95% crushed natural quartz + 5-10% polymer resin |
| Thickness | 15 mm, 20 mm (standard), 30 mm (premium) |
| Hardness | Mohs 6.5-7 |
| Heat Resistance | Moderate — up to 150°C; use trivets |
| Stain Resistance | Excellent; non-porous surface |
| Price Range | ¥600 - ¥1,800 / m² |
| Lifespan | 15-25 years |

Advantages: Non-porous, wide color range, consistent pattern, low maintenance, antibacterial.

Disadvantages: Visible seams, can chip at edges, not suitable for direct high-heat contact.

### 2.2 Granite (花岗岩)

| Attribute | Detail |
| --- | --- |
| Composition | 100% natural igneous rock |
| Thickness | 20 mm, 30 mm |
| Hardness | Mohs 6-7 |
| Heat Resistance | Excellent — withstands direct hot pans |
| Stain Resistance | Good when sealed; porous without sealing |
| Price Range | ¥800 - ¥3,000+ / m² |
| Lifespan | 25+ years (effectively permanent) |

Advantages: Unique natural patterns, high heat tolerance, adds property value.

Disadvantages: Requires periodic sealing (every 1-2 years), natural fissures may propagate, heavy (requires sturdy cabinetry).

### 2.3 Stainless Steel (不锈钢)

| Attribute | Detail |
| --- | --- |
| Composition | 304-grade food-safe stainless steel |
| Thickness | 0.8 mm (cladding) to 1.5 mm (solid top) |
| Hardness | N/A — metal surface |
| Heat Resistance | Excellent |
| Stain Resistance | Excellent; hygienic surface |
| Price Range | ¥1,000 - ¥2,500 / m² |
| Lifespan | 20+ years |

Advantages: Seamless integration with sink, completely hygienic, heatproof, modern aesthetic.

Disadvantages: Shows scratches and fingerprints easily, can dent, noisy (pans clank), limited color options.

### 2.4 Quick Selection Guide

| Priority | Recommended Material |
| --- | --- |
| Budget-conscious | Quartz (mid-range) |
| Low maintenance | Quartz or Stainless Steel |
| Cooking-heavy kitchen | Granite (heat) or Stainless Steel |
| Luxury / resale value | Granite (premium slabs) |
| Hygiene-critical (commercial style) | Stainless Steel |
| Consistent aesthetic | Quartz |

## 3. Edge Profiles

| Profile | Description | Best For | Cost Impact |
| --- | --- | --- | --- |
| Straight (直边) | Flat 90-degree edge | Modern, minimal | Baseline |
| Beveled (斜边) | Angled cut at 45 degrees | Transitional | +5-10% |
| Bullnose (圆边) | Fully rounded edge | Family homes, safety | +10-15% |
| Half Bullnose (半圆边) | Rounded top, flat bottom | Balanced look | +8-12% |
| Ogee (S形边) | Decorative S-curve | Traditional, luxury | +20-30% |
| Waterfall (瀑布边) | Vertical drop to floor | Island features | +30-50% |
| Eased (微圆边) | Slightly rounded corners | Everyday use | Baseline +5% |

Installation Note: Edge profiles must be specified before fabrication. Post-fabrication edge changes require re-polishing the entire slab face and are rarely cost-effective.

## 4. Seam Treatment (拼接缝处理)

### 4.1 Seam Placement Principles

- Avoid placing seams directly in front of the sink or cooktop.

- Minimize the number of seams — ideally one per continuous run.

- Locate seams at natural break points (e.g., at a corner or between cabinet sections).

- Never place a seam within 150 mm of a sink cutout edge.

### 4.2 Seam Execution Steps

| Step | Action |
| --- | --- |
| 1 | Dry-fit both pieces; verify alignment and level |
| 2 | Apply color-matched epoxy adhesive to both edges using a notched spreader |
| 3 | Clamp pieces together using seam clamps; align within 0.5 mm tolerance |
| 4 | Remove excess adhesive immediately with a razor blade before curing |
| 5 | Allow adhesive to cure per manufacturer specification (typically 24 hours) |
| 6 | Polish seam area with progressively finer diamond pads (50, 100, 200, 400 grit) |
| 7 | Final seal (if required by material) |

### 4.3 Acceptable Seam Width

| Material | Max Seam Width |
| --- | --- |
| Quartz | 0.5 mm (barely visible) |
| Granite | 0.5-1.0 mm (depends on pattern) |
| Stainless Steel | Welded — no visible seam |

## 5. Sink Cutout (水槽开孔)

### 5.1 Cutout Types

| Type | Description | Pros | Cons |
| --- | --- | --- | --- |
| Top-mount / Drop-in (台上盆) | Sink rests on counter lip | Easy installation, replaceable | Rim collects debris, visible edge |
| Undermount (台下盆) | Sink mounted beneath counter | Clean counter wipe, seamless | More expensive install, weight concern |
| Integrated (一体盆) | Counter and sink fabricated as one piece | Zero seams, hygienic | Expensive, limited styles |

### 5.2 Cutout Dimensions

| Sink Size | Recommended Cutout (mm) | Min Edge Margin (mm) |
| --- | --- | --- |
| Single bowl 600 mm | 560 x 420 | 60 |
| Double bowl 800 mm | 760 x 420 | 80 |
| Farmhouse apron | Custom per model | 40 (front), 60 (sides/back) |
| Bar prep 380 mm | 340 x 340 | 60 |

Critical: Always use the manufacturer’s template or the actual sink for tracing. Subtract 3-5 mm from the template for undermount installations to account for the mounting rim.

### 5.3 Cutout Execution

- Place template on underside of countertop; trace with pencil.

- Drill four 10 mm starter holes just inside the traced corners.

- Use a jigsaw with a diamond or carbide blade to cut along the line.

- Sand cut edges with 200-grit diamond pad.

- For undermount: install clips or epoxy brackets per sink manufacturer.

- Apply a bead of food-safe silicone caulk between sink rim and countertop.

- Allow silicone to cure 24 hours before connecting plumbing.

## 6. Installation Process

### 6.1 Pre-Installation Checklist

- Cabinetry installed, leveled, and secured

- Cabinet top rails or plywood subtop installed (minimum 18 mm plywood)

- Plumbing rough-in complete and pressure-tested

- Electrical outlets positioned per appliance plan

- Countertop template verified against site measurements

- Slabs inspected for cracks, chips, or color mismatches before cutting

- Access path from truck to kitchen is clear (protect floors and doorways)

- All necessary tools on site: shims, silicone, clamps, level, caulk gun

### 6.2 Installation Steps

| Step | Task | Detail |
| --- | --- | --- |
| 1 | Dry fit | Place all pieces without adhesive; verify fit, alignment, and overhang |
| 2 | Level cabinets | Shim as needed; ensure all tops are within 1 mm of level across the full run |
| 3 | Apply adhesive | Run 6 mm beads of silicone or construction adhesive along cabinet top rails in a zigzag pattern |
| 4 | Set slabs | Lower slabs into position using suction cups (for heavy materials); press firmly |
| 5 | Join seams | Follow seam treatment procedure (Section 4.2) |
| 6 | Verify level | Check with 2 m straightedge; maximum deviation 1 mm over 2 m |
| 7 | Install backsplash | Adhere with silicone; caulk top edge |
| 8 | Cut faucet hole | Use hole saw; diameter per faucet spec (typically 32-35 mm) |
| 9 | Caulk perimeter | Apply color-matched silicone along wall and backsplash joints |
| 10 | Final clean | Remove all protective film, adhesive residue, and dust |

### 6.3 Overhang Specifications

| Application | Max Overhang (unsupported) | Support Required Beyond |
| --- | --- | --- |
| Quartz (20 mm) | 300 mm | 300 mm — install corbels or brackets |
| Granite (30 mm) | 350 mm | 350 mm — install corbels or brackets |
| Breakfast bar (bar seating) | 400-450 mm | Steel brackets or cantilever frame |
| Island waterfall | N/A | Full-height cabinet or steel subframe |

## 7. Maintenance Guide

### 7.1 Daily Care

| Material | Cleaning Method | Avoid |
| --- | --- | --- |
| Quartz | Warm water + mild dish soap, soft cloth | Abrasive pads, bleach, oven cleaner |
| Granite | Stone-specific pH-neutral cleaner | Vinegar, lemon juice, ammonia |
| Stainless Steel | Stainless steel cleaner, microfiber cloth | Steel wool, chlorine bleach |

### 7.2 Periodic Maintenance Schedule

| Interval | Action | Material |
| --- | --- | --- |
| Monthly | Inspect seams and caulk lines; re-caulk if lifting | All |
| Every 6 months | Test granite seal: drop water — if it absorbs, reseal | Granite |
| Annually | Reseal granite with penetrating sealer | Granite |
| As needed | Polish out minor scratches with stone polishing compound | Granite |
| As needed | Remove mineral deposits with stone-safe descaler | All |

### 7.3 Damage Repair Guide

| Issue | Repair Method |
| --- | --- |
| Small chip (quartz/granite) | Fill with color-matched epoxy; sand flush; polish |
| Crack (granite) | Professional repair with epoxy resin and reinforcement pins |
| Scratch (stainless steel) | Polish with grain using abrasive pad (320-400 grit); follow grain direction |
| Burn mark (quartz) | Professional resurfacing; may require section replacement |
| Stain (granite) | Poultice: baking soda + water paste, cover with plastic 24 hours, remove |

## 8. Quality Acceptance Criteria

| Item | Standard |
| --- | --- |
| Levelness | ≤ 1 mm deviation over 2 m |
| Seam visibility | Invisible at 600 mm viewing distance (quartz) |
| Overhang tolerance | ±2 mm from specified dimension |
| Cutout edge | Smooth, no chips or roughness |
| Caulk lines | Uniform width, no gaps or breaks |
| Surface | No scratches, chips, or etching visible under normal lighting |

# 美缝施工详解

## 美缝剂类型

| 类型 | 特点 | 价格 |
| --- | --- | --- |
| 白水泥 | 便宜、易脏 | 5-10元/㎡ |
| 勾缝剂 | 防水、颜色多 | 10-20元/㎡ |
| 美缝剂 | 防水防霉、光亮 | 20-50元/㎡ |
| 环氧彩砂 | 质感好、耐久 | 50-100元/㎡ |
| 聚脲美缝 | 不发黄、耐久 | 80-150元/㎡ |

## 施工流程

### 第一步：清理砖缝

- 用清缝刀清理砖缝

- 深度≥3mm

- 清理灰尘

### 第二步：打美缝剂

- 装入胶枪

- 沿砖缝均匀打入

- 速度均匀

### 第三步：刮平

- 用压缝球或刮板

- 刮平美缝剂

- 清理余料

### 第四步：养护

- 养护24小时

- 避免踩踏

- 避免水接触

## 施工要点

- 施工温度 5-35°C

- 砖缝必须干燥

- 深色砖选深色美缝剂

- 浅色砖选浅色美缝剂

## 常见问题

| 问题 | 原因 | 处理 |
| --- | --- | --- |
| 脱落 | 砖缝不干净 | 重新清理 |
| 开裂 | 填不满 | 重新施工 |
| 发黄 | 质量差 | 选择品牌产品 |

# 泥瓦工施工工艺流程

## 砌筑施工

### 施工流程

- 放线 → 确定墙体位置

- 植筋 → 新旧墙体连接处植筋

- 砌筑 → 砂浆饱满，灰缝8-12mm

- 抹灰 → 墙面找平

- 养护 → 洒水养护≥7天

### 工艺要点

- 砌筑前砖块浇水湿润

- 砂浆配合比符合设计

- 每天砌筑高度≤1.8m

- 门窗洞口上方设过梁

## 防水施工

### 施工流程

- 基层处理 → 清理、修补、做圆弧

- 底涂 → 涂刷底涂一遍

- 面涂 → 涂刷面涂2-3遍

- 闭水 → 蓄水24小时试验

- 保护层 → 防水层上做保护层

### 工艺要点

- 管根、阴阳角加强处理

- 每遍涂刷方向垂直

- 防水层厚度≥1.5mm

- 厨卫墙面防水高度达标

## 贴砖施工

### 施工流程

- 基层处理 → 清理、找平、刷界面剂

- 弹线排砖 → 确定铺贴方案

- 涂抹粘结剂 → 齿形刮板刮涂

- 铺贴 → 橡皮锤敲实

- 勾缝 → 24小时后勾缝

### 工艺要点

- 陶质砖泡水2小时以上

- 墙砖压地砖工艺

- 砖缝均匀一致

- 空鼓率≤5%

## 验收要点

- 墙面垂直度≤3mm/2m

- 表面平整度≤4mm/2m

- 空鼓率≤5%

- 防水闭水试验合格

- 砖缝均匀

- 阴阳角方正

# 瓷砖铺贴详解

## 铺贴方式对比

| 方式 | 特点 | 损耗率 | 适用场景 |
| --- | --- | --- | --- |
| 直铺 | 整齐规整 | 5% | 大部分场景 |
| 斜铺 | 视觉延伸 | 10% | 美式、欧式 |
| 人字铺 | 造型感强 | 15% | 法式、北欧 |
| 工字铺 | 错落有致 | 8% | 工业风 |
| 拼花 | 装饰性强 | 20% | 玄关、背景墙 |

## 铺贴工艺流程

### 1. 基层处理

- 清理基层

- 找平

- 刷界面剂

### 2. 排砖

- 弹线定位

- 预排砖

- 确定切割位置

### 3. 铺贴

- 涂抹瓷砖胶（齿形刮板）

- 铺贴

- 橡皮锤敲实

- 十字定位器控制砖缝

### 4. 勾缝/美缝

- 24小时后进行

- 清理砖缝

- 填入填缝剂或美缝剂

## 美缝施工

### 美缝剂类型

| 类型 | 特点 | 价格 |
| --- | --- | --- |
| 白水泥 | 便宜、易脏 | 5-10元/㎡ |
| 勾缝剂 | 防水、颜色多 | 10-20元/㎡ |
| 美缝剂 | 防水防霉、光亮 | 20-50元/㎡ |
| 环氧彩砂 | 质感好、耐久 | 50-100元/㎡ |

### 美缝施工要点

- 砖缝清理干净

- 填入美缝剂

- 刮平

- 清理余料

- 养护24小时

## 常见问题处理

| 问题 | 原因 | 处理 |
| --- | --- | --- |
| 空鼓 | 粘结不牢 | 拆除重铺 |
| 崩瓷 | 切割不当 | 更换 |
| 色差 | 不同批次 | 同区域同批次 |
| 开裂 | 基层沉降 | 修补 |

# 油漆工艺详解

## 墙面处理

### 基层处理

- 清理墙面

- 修补裂缝

- 刷界面剂

### 批刮腻子

- 第一遍粗刮（≤2mm）

- 第二遍细刮（≤1mm）

- 第三遍局部修补

- 干后打磨

### 打磨

- 240目以上砂纸

- 灯光照射检查

- 平整度≤2mm/2m

## 涂料施工

### 底漆

- 封闭基层

- 增强附着力

- 必须刷

### 面漆

- 第一遍均匀涂刷

- 间隔4-6小时

- 第二遍方向一致

### 施工条件

- 温度5-35°C

- 湿度≤85%

- 通风良好

## 验收标准

- 无流坠、透底

- 颜色均匀

- 表面光滑

- 阴阳角顺直

# 油漆工施工工艺流程

## 墙面施工

### 施工流程

- 基层处理 → 清理、修补、打磨

- 批刮腻子 → 第一遍粗刮，第二遍细刮

- 打磨 → 腻子干后打磨平整

- 刷底漆 → 封闭基层

- 刷面漆 → 第一遍，干后刷第二遍

### 工艺要点

- 每遍腻子干透后再刮下一遍

- 打磨后用灯照检查平整度

- 底漆必须刷，增强附着力

- 面漆两遍方向一致

- 施工温度 5-35°C

## 木器漆施工

### 施工流程

- 基层处理 → 打磨、清理

- 刷底漆 → 封闭木材

- 修补 → 补腻子修补缺陷

- 打磨 → 细砂纸打磨

- 刷面漆 → 2-3遍

### 工艺要点

- 木器漆分开放型和封闭型

- 开放型保留木纹

- 封闭型覆盖木纹

- 每遍干透后再刷下一遍

## 壁纸/壁布施工

### 施工流程

- 基层处理 → 平整、干燥、清洁

- 刷基膜 → 封闭基层

- 裁剪 → 对花裁剪

- 涂胶 → 背面涂胶

- 铺贴 → 从上到下刮平

- 修边 → 裁去多余部分

### 工艺要点

- 壁纸提前 24 小时放室内适应温湿度

- 对花准确

- 接缝严密

- 无气泡、无褶皱

## 验收要点

- 墙面无色差

- 无流坠、透底

- 表面光滑

- 阴阳角顺直

- 壁纸接缝不明显

- 木器漆均匀